Technical and Economic Design of Three Phase Induction motor Using The  Multicriterion Optimization Method

Haider Hussein Kadhim

Babylon University/ Faculty of Engineering

Department of Electrical Engineering

Abstract

This  paper describes the optimum design of three phase induction motor for which three objective function regarding motor efficiency, starting torque and temperature rise as objective function. Multicriterion optimization technique is applied to finding the  optimal solution. The proposed method implemented on a test motor and the results are compared with the, Conventional  design and Simulated Annealing technique(SA) method and also compared with the original motor dimensions . The results show that the Multicriterion Optimization  method (MCDM) gave better result

الخلاصة

هذا البحث يصف التصميم المثالي للمحرك الحثى ثلاثي الأطوار والذي يتضمن ثلاثة أهداف وظائفي فيما يتعلق بالمحرك الكفاءة و العزم وارتفاع درجة الحرارة. الطريقة المقترحة جربت على أحد  المحركات و قورنت مع الطريقة التقليدية  وطريقة (  تقنية التَلْدين المُقَلَّدةِ    ) وكذلك قورنت النتائج مع إبعاد المحرك الاصلى النتائج بينت أن طريقة التعدد المعياري أعطت أفضل النتائج  (SA)

- Introduction

Interest  in means of optimization of electric machine design is high because of  increased cost of electrical energy and pressures for its conservation, plus the increased competition in world markets. The objective of the optimization process is usually to minimize the cost of the machine or to maximize the efficiency of the machine. This paper is concerned with the optimization of three different objective functions namely efficiency, Starting torque and temperature rise.

Three phase Induction motors are the most widely used in domestic, commercial and various industrial applications. Particularly, the squirrel cage rotor type is characterized by its simplicity, robustness and low cost [1], which has always made it very attractive, and it has therefore captured the leading place in industrial sectors. As a result of its extensive use in the industry, induction motors consume a considerable percentage of the overall produced electrical energy. The minimization of electrical energy consumption through a better motor design becomes a major concern. In means of optimization of electric machine design is high because of increased cost of electrical energy and pressures for its conservation, plus the increased competition in world markets. The objective of the optimization process is usually to minimize the temperature rise or to maximize the efficiency of the machine. In the design of induction motors, as in all design problems, the engineer is faced with making a decision in the face of competing objectives[1-3]. The design problem is considered as an optimization process by which the designer seeks the best  design. In certain approaches, the optimization is achieved by applying the principles of nonlinear programming. In stating  a nonlinear programming problem, the basic variables are to be defined and an objective function is to be minimized (or maximized) subjected to non violated constraints[4]. The objective function is a scalar when the designer is interested in achieving one goal. These are many papers, [2-7] that represent the design problem of an induction motor (IM) as a scalar nonlinear programming problem. In the applications of nonlinear programming techniques to optimize the design of IM the objective function may be the annual cost, the material cost, the power factor, or the weight as in airplanes, or a certain performance parameter. In such cases the designer deals with the objective function in a scalar form. In fact, the designer of an induction motor, as in all design problems, is interested in achieving more than one goal. In such case the objective function is defined as a vector function consisting elements each one representing a goal. The goals may be given equal or different weights according to the design requirements[5,7,8.9]. The functions are non-commensurable and usually some of them are in conflict with others. In general there is no single solution which gives the best values for all functions. But the designer will concern on a set of solutions known as non-dominant (non-inferior, Pareto) set. Where he can select the preferred one which achieves his goal to large extent and this design is considered as the optimal solution for the problem. In this case the problem is known as a multicriterion optimization problem. The available papers to the authors, which represents the design problem of a three-phase IM as a multicriterion optimization problem, is that due to [9-16]. In this study the idea of multicriterion optimization approach is applied to the design of three phase induction motor. The multicriterion optimization problem is defined as a multiple criteria decision making (MCDM) problem. In this study the design of a three-phase induction motor is represented as a multicriterion optimization problem with three  objective functions, which are full loud efficiency. temperature rise and starting torque.

2. MATHEMATICAL FORMULATION OF THE DESIGN PROBLEM :

A general nonlinear programming problem can be stated in mathematical terms and the multicriterion optimization problem can be stated: as follows[ ]:

where ,  , and  is the feasible set of problem (1) which is described by inequalities as follows:

Furthermore, in the considered case, we have some additional features.

1) An analytical representation of

is not available and, hence, no derivative information can be used in the solution process.

2)The constraints                                                               are not restrictive in that it is relatively

easy to produce afeasible point and remain in the feasible region. We

denote by the vector made up of all the objective functions, that is,

An ideal solution of (1) would be a point    such that

Unfortunately, such a point   seldom exists, therefore (1) turns into finding some or all the Pareto optimal solutions, that is, points satisfying the following definition.

A point   is a Pareto-optimal solution of (1) if there does not exist any feasible      point  such that

and

for at least one index .   There exists a wide variety of methods that can be used to compute Pareto optimal points. A widely used technique consists of reducing the multiobjective problem (1) to a single-objective one by means of so-called

“secularization” procedure. A first choice consists of transforming (1) into a sequence of constrained problems in which a particular objective function is minimized and the remaining ones are “constrained” to keep their values below a prefixed upper bound. A second choice consists of minimizing a function which is a “combination” of all the objective functions of (1). In this paper, we adopt the second strategy since the first one would require an efficient derivative-free algorithm for a global optimization problem with a feasible set complicated by the presence of hard constraints (namely, those involving the bounds on the objective functions).We employ three different secularization techniques. The first one consists of assigning each objective function

a cost coefficient and then minimizing the function obtained by summing up all the objective functions scaled by their cost coefficients, that is

The second method consists of choosing an “ideal optimal value” for each objective function and then minimizing the Euclidean distance between the actual vector of objective functions and the vector made up of the ideal values, that is

Where  stands for the the  ideal value. The third strategy is similar to the second one in that we consider again the differences ,  , but this time we consider the following problem:

It is possible to prove that each global minimum point of problems (3), (4), and (5) is a Pareto-optimal solution for (1) (see [3]). Obviously, the global solutions of problems (3)–(5) are affected by the coefficients and the ideal values

Naturally, the choice of these values can be done by exploiting any possible a priori knowledge on the underlying optimal design problem. In this paper, we decided to determine these quantities without requiring any a priori information. For this reason, we first solve global optimization problems, namely

Then, we set

3.Applying (MCDM )Optimization to the design of Induction machine

The design optimization of electric motors requires a particular attention in the choice of the objective function that usually concerns economic or performance features. For this reason, we have chosen three conflicting objective functions that can affect the design optimization of three phase induction motors. Particularly:. The following steps were followed in the implementation.

3.1 Problem  Formulation

Machine design optimization is not only running of a mathematical optimization iterative procedure. There are some other important steps, which are required to be followed before the implementation of the  optimization technique. Selection of objective functions, variables and constraints are the main steps. A problem in the selection of variables is that the design problem of IM would have been very much complicated using too many variables.

3.1 A. Variables  The following quantities are chosen as the principle design variables for the optimization

| item | Stator bore diameter | Average air gap flux density | Stator current density | Air gap length | Stator slot depth | Stator slot width | Stator core depth | Rotor slot depth | Rotor slot width |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| symbol | X1 | X2 | X3 | X4 | X5 | X6 | X7 | X8 | X9 |

3.1 B. Constraints

To make a motor practically feasible and acceptable, the constraints have a big role in it. The constraint which gets most effected with the variation in the objective function should be considered with special care. The constraints (C1…….C9) imposed into induction motor design in this paper is as follows which are expressed in terms of variables

| item | temperature rise | full load efficiency | starting torque | no load current | maximum torque | slip, pu | full load power factor | Stack length | maximum stator flux density |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| symbol | C1 | C 2 | C 3 | C 4 | C 5 | C 6 | C 7 | C 8 | C 9 |

.

3.1 C. Objective Functions

Three different objective functions  are considered while designing the machine using optimization algorithms. The objective functions are,

F(x) = C1; Minimization of temperature rise

F(x) = C2; Maximization of Efficiency

F(x) = C3; Maximization of Starting torque

a good design should represent the right compromise among different objectives but the problem consists in searching this “compromise.” The only tool able to solve this problem is represented by the (MCDM) approach .the following multicriterion formulations:

This approach allows us to investigate how each single-objective and multicriterion problem affects the results in terms of performance and independent variables and, above all, allows us to have a wide range of alternative designs among which the designer can choose a better solution

4.Optimization algorithm and design procedure

The optimal design algorithm is applied to the design of three phase induction motor for witch three objective functions regarding motor efficiency, temperature rise and starting torque are used. And the dimensions, parameters and characteristics of the optimally designed motor are compared with those of prototype one. The optimization problem is solved using weighting min-max algorithm. The proposed method approach for the optimal design of induction motors requires us to find global solutions. This algorithm, which belongs to the class of Controlled Random Search algorithms, is based on the following points

1) Initialization: The objective function is sampled on a set points randomly chosen within the feasible set . 2) Stopping criterion: If the maximum and minimum value of the objective function over are sufficiently close to each other, the algorithm stops. 3) Search phase: points are randomly chosen on set . Then, by exploiting as much as possible the information on the objective function conveyed by these points, a new point is produced. 4) Updating phase: If the objective function value on the new point is better then the maximum function value, then set is updated by adding the new point and discarding the worst one. The algorithm continues iterating

The problem solution is achieved by suggesting a design, which takes in consideration the three goals. The approach of MCDM is applied to optimize the design of a three- phase IM, which is already in production by Sate Electrical Industries Company (SEICO). The original motor dimensions are given in (Appendix-1-B-motor-2).These values represent the starting   (initial) values of basic variables in the optimization procedure.

The design procedure is achieved by representing the motor by its equivalent circuit fig (1) based on the revolving magnetic field theory.. The. equivalent circuit approach will give results, which are close to the test results if the necessary modifications are taken into account. These modifications are due to skew, saturation and skin effects. The iron, friction

and winding losses are taken into consideration(V.P. Sakthivel, R. Bhubaneswar and S. Subramanian ,2010).

Fig(1)The per-phase equivalent circuit model of induction motor

5. Results And Discussion

The modified Multicriterion algorithm is a solution method to the vector optimization problem. The algorithm  searches both the noninferior solution set and the best compromise solution. The. optimal design results are given in tables. Which shows the solution corresponding to the minimum of each objective function. The best compromise solution with the prototypes is given in tables.The optimally designed motor shows higher efficiency, Starting Torque and minimum in   Temperature rise. The optimization results suggest the following comments1 .

1.The minimization of the single objective function, gives rise to a significant improvement of the chosen term, but affects heavily the other ones. Particularly,

the optimization of motor efficiency allows to reach a very interesting result  that permits to label the motor as “high efficiency motor” according to the new European classification scheme [5].

2.(MCDM) Optimization method is implemented to optimize the design of induction motor whose specifications are available in appendix. The results of proposed method design and their comparison with( SA ) and normal design are given in the Tables 1,2, 3, 4 and 5..

3.When (MCDM) considered efficiency of the motor as an objective function table(2) the resulting design gave considerably better results than normal design and also quite better than (SA)method. Temperature rise and slip are lower in (MCDM ) but main dimensions are higher than other methods.

4.For Starting torque maximization ,table(3) also, (MCDM) Optimization method offers better results than others  significantly. In this case, main dimensions are higher but temperature rise considerably reduced. Full load slip in (MCDM) Optimization method is smaller than normal design and SA.

5. For temperature rise minimization table(4), again (MCDM) Optimization method performed well which improvement percentage is 18.37%, 4.57% compared to normal design and( SA )method. Here main dimensions are lower and efficiency is slightly better than others. For over all performance (MCDM) Optimization method gave good results than others so that it can be used for design optimization of induction motor.

6.From the design results presented in tables (2) and(4), particularly the following variables are dominated in (MCDM) Optimization method to get optimum value of all objective functions;

1.Width of the rotor slot

2.Depth of the rotor slot

3.Length of the stator stack

Table(1):Optimum Design Results For Minimization of temperature rise

| Objective function | Objective function | Objective function | Conventional design | Variables |
| --- | --- | --- | --- | --- |
| Starting Torque | Temperature Rise | Efficiency |  |  |
| 0.00555 | 0.00457 | 0.00433 | 0.00132 | Width of the stator slot (m) |
| 0.02118 | 0.02258 | 0.01515 | 0.021 | Depth of the stator slot (m) |
| 0.00454 | 0.00374 | 0.00355 | 0.0068 | Width of the rotor slot (m) |
| 0.00291 | 0.00499 | 0.00660 | 0.0093 | Depth of the rotor slot (m) |
| 2.00 | 1.632 | 2.00 | 0.6 | Air gap flux density (wb/m2) |
| 0.0005 | 0.0005 | 0.0005 | 0.0003 | Air-gap length (m) |
| 0.0505 | 0.0536 | 0.0416 | 0.0699 | Full load slip |
| 0.0999 | 0.189 | 0.0890 | 0.105 | Stator bore diameter (m) |
| 0.2179 | 0.099 | 0.1913 | 0.181 | Stator outer diameter (m) |
| 0.114 | 0.114 | 0.109 | 0.125 | Stack length (m) |
| 41.810 | 38.209 | 39.83 | 46.8178 | Temperature rise, °C |
| 0.825 | 0.814 | 0.8356 | 0.80309 | Efficiency % |
| 4.966 | 3.133 | 3.730 | 1.2027 | Starting torque, pu. |
| 0.813 | 0.858 | 0.800 | 0.8041 | Power factor |

Table (2):Optimum Design Results For Efficiency Maximization

| Proposed method | (SA) method | Conventional design | Items |
| --- | --- | --- | --- |
| 0.004223 | 0.0011 | 0.00132 | Width of the stator slot (m) |
| 0.014815 | 0.0159 | 0.021 | Depth of the stator slot (m) |
| 0.003615 | 0.005 | 0.0068 | Width of the rotor slot (m) |
| 0.006599 | 0.0091 | 0.0093 | Depth of the rotor slot (m) |
| 1.4320 | 0.521 | 0.6 | Air gap flux density (wb/m2) |
| 0.00046 | 0.0003 | 0.0003 | Air-gap length (m) |
| 0.0416 | 0.056 | 0.0699 | Full load slip |
| 0.0890 | 0.102 | 0.105 | Stator bore diameter (m) |
| 0.1913 | 0.177 | 0.181 | Stator outer diameter (m) |
| 0.1109 | 0. 097 | 0.125 | Stack length (m) |
| 39.713 | 391.41 | 46.8178 | Temperature rise, °C |
| 0.84856 | 0.82848 | 0.80309 | Efficiency % |
| 3.73120 | 1.3444 | 1.2027 | Starting torque, pu. |
| 0.82100 | 0.8333 | 0.8041 | Power factor |

Table (3):Optimum Design Results For Starting Torque Maximization

| Proposed method | SA method | Conventional design | Items |
| --- | --- | --- | --- |
| 0.004955 | 0.0012 | 0.00132 | Width of the stator slot (m) |
| 0.0211 | 0.0187 | 0.021 | Depth of the stator slot (m) |
| 0.00444 | 0.0056 | 0.0068 | Width of the rotor slot (m) |
| 0.00293 | 0.0071 | 0.0093 | Depth of the rotor slot (m) |
| 1.4320 | 0.4713 | 0.6 | Air gap flux density (wb/m2) |
| 0.0005 | 0.0004 | 0.0003 | Air-gap length (m) |
| 0.05 | 0.0645 | 0.0699 | Full load slip |
| 0.0989 | 0.1028 | 0.105 | Stator bore diameter (m) |
| 0.21709 | 01733 | 0.181 | Stator outer diameter (m) |
| 0.1144 | 0.1162 | 0.125 | Stack length (m) |
| 41.610 | 64.475 | 46.8178 | Temperature rise, °C |
| 0.8365 | 0.79179 | 0.80309 | Efficiency % |
| 4.9776 | 1.3776 | 1.2027 | Starting torque, pu. |
| 0.813 | 0.7938 | 0.8041 | Power factor |

Table (4):Optimum Design Results For Temperature Rise Minimization

| Proposed method | SA method | Conventional design | Items |
| --- | --- | --- | --- |
| 0.00357 | 0.0013 | 0.00132 | Width of the stator slot (m) |
| 0.02226 | 0.0236 | 0.021 | Depth of the stator slot (m) |
| 0.003644 | 0.005 | 0.0068 | Width of the rotor slot (m) |
| 0.00499 | 0.0093 | 0.0093 | Depth of the rotor slot (m) |
| 1.2632 | 0.439 | 0.6 | Air gap flux density (wb/m2) |
| 0.000501 | 0.0004 | 0.0003 | Air-gap length (m) |
| 0.0526 | 0.0684 | 0.0699 | Full load slip |
| 0.169 | 0.101 | 0.105 | Stator bore diameter (m) |
| 0.0989 | 0.171 | 0.181 | Stator outer diameter (m) |
| 0.113 | 0.1216 | 0.125 | Stack length (m) |
| 38.19209 | 40.0391 | 46.8178 | Temperature rise, °C |
| 0.8134 | 0.803748 | 0.80309 | Efficiency % |
| 3.14033 | 1.117 | 1.2027 | Starting torque, pu. |
| 0.8658 | 0.7814 | 0.8041 | Power factor |

Table (5):Comparison .(MCDM) Optimization method with normal design and SA method

| Test Motor |  |  | Objective Function |
| --- | --- | --- | --- |
| Proposed method | SA Method | normal |  |
| 0.52 | .86 | 4.04 | F(x) = A |
| 102.45 | 260.48 | 312.9 | F(x) = B |
| 9.43 | 4.57 | 18.37 | F(x) = C |

In order to reach the equilibrium optimization point of the induction motor the Efficiency , Total losses and Torque is expressed as a function of the values motor dimensions (Equations below)

f(X)=0.0003X5−0.0155X4+0.3483X3−3.8381X2+21.497X+31.828

Equation (1)Efficiency versus depth of the rotor slot in the optimized motor

f(X)=2E−09X6−1E−06X5+0.0004X4−0.069X3+5.9075X2−258.33X+4500.8

Equation (2) Efficiency versus stator stack length in the optimized motor

f(X)=0.2438X6−6.4433X5+68.077X4−363.72X3+1013X2−1322.8X+615.86

Equation (3)  Efficiency versus width of the stator slot in the optimized motor

f(X)=−34.894X6+902.14X5−9270.9X4+47795X3−126998X2+155350X−55367

Equation (4) Total losses versus width of the stator slot in the optimized motor

f(X)=0.0017X5−0.07X4+1.1417X3−8.9X2+33.007X−31.48

Equation (5)Total losses versus depth of the rotor slot in the optimized motor

f(X)=6E−08X6−5E−05X5+0.0153X4−2.6622X3+268.48X2−14828X+349746

Equation (6)Total losses versus stator stack length in the optimized motor

f(X)=0.0017X5−0.07X4+1.1417X3−8.9X2+33.007X−31.48

Equation (7)Torque versus width of the stator slot in the optimized motor

f(X)=−8E−10X6+5E−07X5−0.0001X4+0.0218X3−1.7694X2+76,222X−1348.9

Equation (8)  Torque versus stator stack length in the optimized motor

6. Conclusions

This paper investigated the optimal design of induction motor using MCDM with three objective functions, efficiency, starting torque and temperature rise. The weighting min-max method is used to solve this problem It is concluded offered good results compared with normal design and SA method  it is more suitable to design optimization of induction motor.

7.References

V.P. Sakthivel, R. Bhubaneswar and S. Subramanian, "Adaptive Particle Swarm Optimization for the Design of Three-Phase Induction Motor Considering the Active Power Loss Effect , International Journal of Computer and Electrical Engineering, Vol. 2, No. 4, August, 2010 ,1793-8163

Appelbaum,J., Fuchs,E.E. and White,J.C Optimization of three-phase induction motor design ,part-I: formulation of the optimization technique" IEEE trans.,EC-,(3),pp. 407-414, 1987

Appelbaum,J., Kahn,I.A., and Fuchs,E.E.: Optimization of three-phase induction motor design ,part-II: The efficiency and cost of an optimal design" IEEE trans. on Energy Conversion,EC-2,(3),pp. 415-422, 1987.

R. Bhuvaneswari, S. Subramanian, “Optimization of Three-Phase Induction Motor Design using Simulated Annealing Algorithm”, Electric Power omponents and Systems, Vol. 33, 2005, pp. 947-956.

Fei,R., Fuchs,E.F., and Huang,H., "Comparison of two optimization techniques as applied to three-phase induction motor design" IEEE Trans. on Energy Conversion EC-4,(4),Dec. ,pp.651-660, 1989..

Fetih,N.H.,and El-Shewy H.M. "Induction motor optimum design including active power loss effect" IEEE Trans. EC -4,(3),Sept.,pp.155-160, 1986

Jazdzynski,W.: "Multicriterion optimisation of squirrel -cage induction motor design" IEE proceedings, pt.-B, vol-136,(6), pp 299-307,1989.

Kannan, R. ; Bhuvaneswari, R.  and Subramanian S. ,  Optimal Design of Three-phase Induction Motor Using Particle Swarm Optimization VOL. 6, NO. 2, UMMER-FALL 2007

Ashish Ghosh " Evolutionary Algorithms for Multi-Criterion optimization: Aurvey."International Journal of computing Sciences" Vol.2,No.1 April 2009

Mohammed and al-uzri Multicriterion optimization approach for the design  of induction motor. Engineering journal of the university of Qatar, Vol, 12, 1999, pp.151-164

Ramarathnam,R., and Desai,B.G.: "Optimization of poly phase induction motor design, a nonlinear programming approach "IEEE Tran. on PAS-90,(2), pp570-578,1971.

Ramarathnam,R., B.G.Desai and V. Subba Rao, "A Comparative Study of Minimization Techniques for Optimization of Induction Motor Design", IEEE Trans., PAS-92, (5), Sept. /Oct. pp. 1448-1454, 2010.

Singh,B., Singh,B.P, Murthy,S.S., and Jha,C.S "Experience in design optimization of induction motor using "SUMT' algorithm" IEEE Tran. PAS-102, (10), pp.3379-3384, 1983.

R. Bhuvaneswari, S. Subramanian, “Optimization of Three-Phase Induction Motor Design using Simulated Annealing Algorithm”, Electric Power omponents and Systems, Vol. 33, 2005, pp. 947-956.

W. Jazdzynski, Deng " optimisation of squirrel-cage induction motor design" IEE PROCEEDINGS, Vol. 136, Pt. B, No. 6, NOVEMBER 1989

I. Mergias, K. Moustakas ∗, A. Papadopoulos, M. LoizidouMulti-criteria decision aid approach for the selection of the best compromise management scheme for ELVs: The case of Cyprus Journal of Hazardous Materials 147 (2007) 706–717

B. Mirzaeian, M. Moallen, V. Tahani, and C. Lucas, “A newmulti-objective  optimization method based on genetic algorithm and its application for optimal design of a switched reluctance motor,” Proc. ICEM 2000, vol. 3, pp. 1675–1678

Appendix-1-A

Specification of Test Motor [Kannan, et al., 2007]

Capacity 3 hp

Voltage per phase 400 volts

Frequency 50 Hz

Number of poles 4

Number of stator slots 36

Number of rotor slots 44

Y-connected, squirrel cage rotor

Appendix-1-B      (original motor dimensions)

| 0.125 | Stack length (m |
| --- | --- |
| 0.00132 | Width of the stator slot (m) |
| 0.021 | Depth of the stator slot (m) |
| 0.0068 | Width of the rotor slot (m |
| 0.0093 | Depth of the rotor slot (m) |
| 0.6 | Air gap flux density (wb/m2) |
| 0.0003 | Air-gap length (m) |
| 0.0699 | Full load slip |
| 0.105 | Stator bore diameter (m) |
| 0.181 | Stator outer diameter (m) |
| 0.8041 | Power factor |
| 46.8178 | Temperature rise, °C |
| 0.80309 | Efficiency % |
| 1.2027 | Starting torque, pu. |