Determination some Heavy metals in Sediments of Shatt Al-Hilla River by Using Modified Single Chemical Fractionation Technique

Hazim Aziz Hamza AL-Robai

College of Ecology AL-Qasim Green University

Abstract

Surfacial sediment composite samples were collected from six main sampling stations at Shatt Al-Hilla River from Khafaja area to Betta bridge in Babylon province.

A four-step modified single sequential extraction procedure was used to determine heavy metal (Pb,Ni,Fe and Cu) chemical phases in the sediment samples.

The results showed that there was concentrated for the high values of total concentrations of studied heavy metals in the station (5). When comparing the percentage of heavy metals in exchangeable and carbonate fractions at studied stations with risk assessment code we noted that the Ni,Fe and Cu were located within the degree: No risk. while Pb metal was within the degree: Low risk which indicates the dominance of anthropogenic sources effect.

الخلاصة

جمعت عينات مركبة  لرواسب سطحية من ستة محطات رئيسة من شط الحلة من منطقة خفاجة الى جسر بتة عند مركز محافظة بابل .

استخدمت طريقة استخلاص متتالي منفردة محورة باربعة خطوات لغرض تقدير الصور الكيميائية للعناصر الثقيلة( Pb  و Ni وFe وCu) في عينات الرواسب.

اظهرت النتائج وجود تركيز للقيم المرتفعة للعناصر الثقيلة الكلية في المحطة رقم(5).وعند مقارنة النسب المئوية للعناصر الثقيلة في الجزئين المتبادل والمرتبط مع الكربونات في المحطات المدروسة مع شفرة تقدير الخطر لوحظ بان عناصر Ni و Fe و Cu كانت واقعة ضمن الدرجة: لا خطر الآمنة بيئيا. في حين كان عنصر Pb ضمن الدرجة :خطر قليل وهي اشارة لسيادة تاثير مصادر النشاطات البشرية.

Introduction

Heavy metals in aquatic systems are removed from the water column by interacting with particulate or organic matter, and are deposited as sediments (Puig et al.,1999; Tang et al.,2008).

It is essential to distinguish and quantify the various forms of metals to yield a better understanding of potential and actual environmental impacts of contaminated sediments (Sakan et al.,2007). Sediment constitutes the most important sink of metals and other pollutants, it can act as a nonpoint source and have the potential to release the sediment-bound metals and other pollutants to overlying waters, and in turn adversely affects aquatic organisms (Wang et al.,2004)

In the bottom sediments, the distribution of heavy metals is affected by mineralogical and chemical composition of suspended material, anthropogenic influences, and in situ processes such as deposition, sorption, enrichment in organism, etc. (Forstner and Muller,1975; Jain et al., 2005)

The behavior of the elements in the environment (bioavailability, toxicity, and distribution) cannot be reliably predicted assuming their total concentration (Alomary and Belhadj 2007). The potential environment risk of heavy metal in sediments depends on both its total content and speciation. In fact heavy metals have different chemical forms in sediments (Jing et al.,2009).  Sequential extraction is an important and widely applied tool for gaining information on potential mobility (hence, potential bioavailability and toxicity) of potentially toxic metals in the environment (Dubravka et al.,2010).

The aim of this paper is to study the sediments pollution with different types of heavy metals found along with the chemical forms and to assess their toxicity  in the river by using the risk assessment code.

Material  and Methods

Sediment sampling

Surfacial sediment composite samples were collected from six main sampling stations at Shatt Al-Hilla River from Khafaja area to Betta bridge in Babylon province .All samples were transferred in labeled bags. Sediment samples were air-dried, ground, passed through a 2mm- sieve, and carefully preserved until laboratory analysis.

characteristics of Sediments

Sediments pH and EC were measured in 1:2 sediment water suspension (Jackson, 1973). Organic matter content was determined following modified Walkley and Black,s method (1936) as described by Jackson (1973).Total Carbonate in sediment samples was measured by acid neutralization method as described by Schollenberger (1945).The proportions of sand, silt and clay-sized particles were determined according to (Day, 1965).

Modified single sequential extraction technique

A four-step sequential extraction procedure was used to determine heavy metal fractions(chemical phases) in the sediment (Badri and Aston, 1983;Ismail and Ramli, 1997)The method is modified and designed to separate the heavy metal into four fractions.

(1) Fraction 1: Easily, freely or leachable and exchangeable (EFLE). About 5 g of sample was continuously shaken for 3 h with 25 ml 1.0M ammonium acetate (NH4CH3COO), pH 7.0 at room temperature.

(2) Fraction 2: metals bound to carbonate. 5 g of sample was continuously shaken for 5 h with 25 ml 1.0M ammonium acetate (NH4CH3COO), pH 5.0 at room temperature.

(3) Fraction 3: Oxidisable-organic. 5 g of sample was first oxidized with 7.5 ml H2O2 (R&M Chemicals 35%) in a water bath at 90˚C. After cooling, the metal released from the organic complexes was continuously shaken for 3 h with 25 ml of 1.0M ammonium acetate (NH4CH3COO) acidified to pH 2.0 with HCl, at room temperature.

(4) Fraction 4: Residual. Metals in this fraction is mainly ,fixed in the primary and secondary minerals (Wu et al., 2004). 5 g of sample was digested in a 5 ml combination (ratio of 4:1) of concentrated HNO3 (AnalaR grade, R&M Chemicals 65%) and HClO4 (AnalaR grade, R&M Chemicals 70%).

Results and discussion

Sediment characteristics

Table (1) showed the characteristics of sediment under study ,which related to the speciation of studied heavy metals in surfacial sediments. The pH values of the river sediments slightly varies from 7.8 to 8.1 indicating that the sediments were weakly alkaline. The EC values varies from 0.86 to 12.27 dS.m-1 . The maximum value of 12.27 dS.m-1of EC at station 6 indicating the anthropogenic influences on the sediment quality (Zhan et al ,2010) specially agricultural and domestic activities in Hilla city .Organic matter, and carbonate (CaCO3) are of good correlation with the chemical fractions of trace metals. Metals are easy to combine with organic matter and carbonate(Wang et al., 2010). Organic matter, CaCO3, are within the range of 0.16 % to 0.25 % and 23.2 to 32.0%, respectively.

The Results showed that the main component of surfacial sediment was silt + clay, with a range from 52% to 78%. Station 4,2 and 5 had the highest silt + clay contents whereas station 1 and 6 had relative low silt + clay contents.

Table 1. Sampling stations and some characteristics of Surfacial sediments of Shatt Al-Hilla River.

Stations

1

2

3

4

5

6

pH

7.9

7.9

7.8

7.9

8.1

7.9

EC dS..m-1

0.86

1.27

3.22

1.26

1.03

12.27

Carbonate%

23.2

23.8

32.0

25.0

27.7

24.5

Silt +Clay%

52

75

60

73

78

55

O.M %

0.18

0.20

0.25

0.16

0.18

0.22

Heavy Metal Fractionation

Figure (1-8) show the concentrations(in ppm) and percentage(%) of heavy metal (Pb ,Ni, Fe and Cu) chemical fractions in surfacial sediments at study stations respectively.

Lead is a naturally occurring element that has no recognized essential biological function; yet it is toxic to almost all organisms. Recognition of its toxicity appears to date back to Hippocrates, and the clinical effects have been recognized for centuries( Rajeshwa and Ibanez 1997).

In this study, the total concentration(ppm) of Pb is in the range of 18.5 (station 2)to 27.06 (station 5) .In the  fraction 1 ranged from 0.35(station 2)to 2.7(station 5) with percentage of 4.76% at all studied stations. In the fraction 2 ranged from 0.66(station 5) to 1.3(station 6) with percentage of 4.88% . In the Fraction 3 ranged from 5(station 1) to 7.8 (station 6) with percentage of 30.8%. In the fraction 4 ranged from 10(station 3)  to 17.5(station 1) with percentage of 59.48% . The percentage of fraction 1 and 2 at all stations (9.64).

Nickel is generally considered to be an essential trace element. It is subject to homeostatic regulation and mobilization in response to disease and physiological stress, and its absence or lack in the mammalian organism produces well-defined deficiency symptoms (Nieboer et al., 1988).

The total concentration (ppm) of Ni is in the range of 114.5(station3) to 140.5(station 5) .In the fraction 1 ranged from 0.3(station 6) to 0.8(station 2) with percentage of 0.42% .In the fraction 2 ranged from 0.4(station 6) to 4.2(station 1)  with percentage of 1.9%. In the fraction 3 ranged from 9.7(station 6) to 11.2(station 1) with percentage of 8.42%. In the fraction 4 ranged from 100.2(station 3) to127.9(station 5)  with percentage of  89.25%.The percentage of fraction 1 and 2 at all stations (2.35).

Iron is the most important metal and one of the major constituents of the lithosphere. Fe plays a special role in the behavior of several trace elements and is in the intermediate position between macro- and micronutrients in plants,animals and humans) Kabata-Pendias and Arun  2007).

The total concentration (ppm) of Fe is in the range of 629.0(station 4) to 1228.9(station 6) .In the  fraction 1 ranged from 0.2(station 2 and 4) to 1.0(station1) with percentage of  0.04%. In the fraction 2 ranged from 2.7(station5)  to 6.9 (station 2) with percentage of 0.41%. In the fraction 3 ranged from 4.1(station 1) to 29.3(station 6) with percentage of 1.22%. In the fraction 4 ranged from 620.8(station 4) to 1193.6(station 6) with percentage of 98.26%. The percentage of fraction 1 and  2 at all stations (0.45).

Copper is one of the oldest known metals and is the 25th most abundant element in the Earth’s crust. Prior to the identification of copper as a micronutrient, it was regarded as a plant poison .Therefore, no discussion of copper toxicity can rightfully begin without mention of its use as fungicide( Reuther et al.,1966; Barker and David .2007

The total concentration(ppm) of Cu is in the range of 8.21(station 6) to 21.8(station 5) .In the fraction 1 ranged from 0.05(station 4) to 0.23(station 5) with percentage of 0.88%. In the fraction 2 ranged from 0.10(station 1)  to 0.22(station 5) with percentage of 1.25%. In the fraction 3 ranged from 0.3(station 1and 3) to 0.75(station 5) with percentage of 3.8%. In the fraction 4 ranged from 7.2(station 6) to 20.6(station 5) with percentage of 94.06%. The percentage of fraction 1 and 2 at all stations (2.12).

Fig. 1 Concentration (ppm) of lead at study stations

Fig. 2 Chemical fractionation of lead as % at study stations

Fig. 3 Concentration (ppm) of nickel at study stations

Fig.4 Chemical fractionation of nickel as % at study stations

Fig. 5 Concentration (ppm) of iron at study  stations

Fig.6 Chemical fractionation of iron as % at study stations

Fig. 7 Concentration (ppm) of copper at study stations

Fig.8 Chemical fractionation of copper as % at study stations

It is clear from the results of modified single chemical fractionation technique used to determination Heavy metals(Pb,Ni,Fe and Cu) in Sediments of Shatt Al-Hilla River there was concentrated for the high values of total concentrations(the sum of all fractions) of studied heavy metals in the station (5) which is located near the center of Hilla city may be attributed to the anthropogenic activities (terrestrial inputs) such as agricultural and domestic discharges rather than the natural rock weathering effect .

The total concentrations of heavy metals(Pb,Ni,Fe and Cu) at present study are less than those obtained by Al-Taee (1999) when she studied the trace elements in the sediments of Shatt Al-Hilla has due to the stopover of industrial activity in areas of Musayyib and Al-Sadaa in the years followed the previous study due to wars, which reflected on the quantity and quality of solid and liquid loaded with pollutants that spilled in Shatt Al-Hilla.

Recently, it is widely accepted that the role of aquatic sediments as a sink or as a source of pollutants cannot be fully assessed by measuring total metal concentrations, because the chemical form of the metal in the sediment determines behaviour and remobilization in the environment. By studying the distribution of the metals between the different phases, their contamination risk can be found out( Jain et al.,2010).

Jain et al.,(2010) reported that  the evident from the results of the fractionation study that the metals in the sediments are bound to different fractions with different strengths. The strength values can, therefore, give a clear indication of sediment reactivity, which in turn assess the risk connected with the presence of metals in an aquatic environment. Exchangeable and carbonate bound metals are weakly bound to sediment components and may be equilibrate easily with the aqueous phase. and is considered more mobile and dangerous than other fractions(Moalla et al .,1997; Wang et al.,2010).

The risk assessment code indicates the sediment which can release heavy metal in exchangeable and carbonate fractions ; % of the total concentration<1: No risk, will be considered safe for the environment, 1–10 : Low risk, 11–30:medium risk, 31–50 high risk, >50:very high risk, and can easily enter the food chain (Perin et al. 1985):.

When comparing the percentage of heavy metals(Pb,Ni,Fe and Cu) in Exchangeable(fraction 1) and carbonate fractions(fraction 2) at studied stations with risk assessment code we noted that the Ni,Fe and Cu were located within the degree: No risk. while Pb metal was within the degree: Low risk which indicates the dominance of anthropogenic sources (Jain et al.,2010).

References

Alomary, A. A. and Belhadj, S. (2007). Determination of heavy metals (Cd, Cr, Cu, Fe, Ni, Pb, Zn) by ICP-OES and their speciation in Algerian Mediterranean Sea Sediments after a five-stage sequential extraction procedure.Environmental Monitoring and Assessment,135, 265–280.

Al-Taee, Maysoon Mhdi Salleh.(1999)Some Trace Elements in Water,Sediments,Fish and Plants of Shatt Al-Hilla River, PhD thesis .College of Sciences.Babylon Univ.(In Arabic)

Badri, MA. and Aston, SR. (1983). Observations on heavy metal geochemical associations in polluted and non-polluted estuarine sediments. Environmental Pollution Series B,Chemical and Physical; 6: 181-93.

Barker,Allen V. and David J. Pilbeam. (2007).Handbook of Plant Nutrition . Taylor & Francis Group, LLC

Day, P.R. (1965) Particle fractionation and particle – size analysis. In: Black, A. C., Evans D.D., Ensminger, L. E., White, J.L. and Clark, F.E. (eds) Methods of Soil analysis. Part1, pp. 545-566 American Society of Agronomy, Madison, Wisconsim. US.

Dubravka ,Relic;Dragana, Dord evic; Aleksandar, Popovi´c ; Milka, Jadranin .and Predrag Poli´c. (2010).Fractionation and potential mobility of trace metals in Danube alluvial aquifer within an industrialized zone ,Environ Monit Assess 171:229–248.

Forstner, W. and  Muller, G. (1975). Factors controlling the distribution of minor and trace elements (heavy metals V, Li, Sr) in recent lacustrine sediments. Resumes des Publications IX Congress Intl.De Sedimentologie, Nice, Theme II (p. 6).

Jackson, M. L. (1973). Soil chemical analysis. New Delhi: Prentice Hall.

Jain, C. K; Singhal, D. C. and Sharma, M. K. (2004).Adsorption of zinc on bed sediment of River Hindon: Absorption models and kinetics. Journal of Hazardous Materials, B114, 231–239.

Jain, C. K., Singhal, D. C., & Sharma, M. K. (2005).Metal pollution assessment of sediment and water in the river Hindon, India. Environmental Monitoring and Assessment, 105(1–3), 193–207.

Jain ,C. K; V. V. S. Gurunadha Rao ; B. A. Prakash ; K. Mahesh Kumar ;Mitsuo, Yoshida.(2010). Metal fractionation study on bed sediments of Hussainsagar Lake, Hyderabad, India, Environ Monit Assess , 166:57–67.

Jing, Lin;Minggang ,Cai; Anxiang, Qi;Hongyou, Hu; Canrong, Qiu; Yun, Wang and Qingquan, Hong. (2009).Contamination level and speciation of heavy metals in sediments from Yundang Lake, Xiamen, Second International Conference on Environmental and Computer Science.

Kabata-Pendias, Alina and Arun, B. Mukherjee.( 2007). Trace Elements from Soil to Human. Springer Berlin Heidelberg New York.

Ismail A, Ramli R. (1997). Trace metals in sediments and mollusks from an estuary receiving pig farms effluent. Environmental Technology; 18: 509-15.

Moalla, S. M. N; R. M. Awadallah, M. N. Rashed . and M. E. Soltan.(1998). Distribution and chemical fractionation of some heavy metals in bottom sediments of Lake Nasser, Hydrobiologia 364: 31–40,. 31.

Nieboer ,E.; R. T. Tom. and W. E.( 1988). Sanford, Nickel metabolism in man and animals, Nickel and Its Role in Biology (H. Sigel and A. Sigel, eds.), Marcel Dekker, New York, , pp. 91–121.

Perin, G; Craboledda, L; Lucchese, M; Cirillo, R; Dotta, L; Zanette, M. L;et al. (1985). Heavy metal speciation in the sediments of Northern Adriatic sea— a new approach for environmental toxicity determination. In T. D. Lekkas (Ed.), Heavy metal in the environment (Vol. 2, pp. 454–456). Edinburgh: CEPConsultants.

Puig, P; A. Palanques, J.A; Sanchez-Cabeza. and P. Masque.( 1999). Heavy metals in particulate matter and sediments in the southern Barcelon a sedimentation system (North-western Mediterranean), Mar. Chem., vol. 63, pp. 311-329.

Rajeshwa, Krishnanr. and Ibanez ,Jorge G.(1997). Environmental Electrochemistry:Fundamentals and Applications in Pollution Sensors and Abatement.Publisher: Elsevier Science & Technology Books.

Reuther , W, C.K. and Labanauskas. (1966). Copper. In: H.D. Chapman, ed. Diagnostic Criteria for Plants and Soils. Berkeley, CA: University of California Division of Agricultural Sciences Press, pp. 394–404.

Sakan ,Sanja; Ivan, Grٌeti and Dragana, worpevi .(2007).Distribution and Fractionation of Heavy Metals in the Tisa (Tisza) River SedimentsEnv Sci Pollut Res ,14 (4) 229 – 236.

Schollenberger, C. J. (1945). Determination of carbonates in soil. Soil Science, 59, 57–63.

Tang, C.W.Y; C.C.M. Ip, G; Zhang, P.K.S; Shin, P.Y; Qian and X.D. Li.(2008). The spatial and temporal distribution of heavy metals in sediments of Victoria Harbour, Hong Kong, Mar. Pollut. Bull., vol. 57, , pp. 816-825.

Wang, H.; C,X Wang; Z.J. Wang and Z.H, Cao.(2004). Fractionation of heavy metals in surface sediments of Taihu Lake, East China Environmental Geochemistry and Health 26: 303–309,. 2004 Kluwer Academic Publishers. Printed in the Netherlands.

Wang ,Lijuan ;Ruilian, Yu ; Gongren, Hu .and Xianglin, Tu.(2010). Speciation and assessment of heavy metals in surface sediments of Jinjiang River tidal reach, southeast of China, Environ Monit Assess 165:491–499.

Wu, P., Liu, C. Q., Zhang, G. P. and Yang, Y. G. (2004).Chemical forms and ecological risks of heavy metals in river sediment at carbonatite mining area. Rural Eco-Environment, 20(3), 28–31.

Zhan ,Shuifen ;Shitao, Peng ;Chunguang, Liu ;Qing, Chang and Jian Xu. (2010).  Spatial and Temporal Variations of Heavy Metals in Surface Sediments in Bohai Bay, North China . Bull Environ Contam Toxicol: 84:482–487.