Heat Transfer Analysis  of Convection and Radiation  in a Triangular Fins Heat Exchanger

Qusay R. Al-Hagag               Hameed   K. Al Naffiey,         Hayder Krady R.

Department of Mechanical Engineering, College of Engineering

University of Babylon

1. Abstract

The effects of convection and radiation on triangular heat exchanger fins of a catalytic reactor are investigated. Heat is transferred by conduction along the fin and dissipated from the surface by convection and radiation. The base of the fin is maintained at a variable elevated temperature, while the tip loses heat due to convection and radiation. Due to symmetry and simplicity of analysis,  one of the walls is considered.

A general finite difference formulation, that accounts for free and forced convection and for radiative heat transfer with a simple model is developed  to predict the temperature and rate of heat transfer along the fin. In this study, the convection heat transfer coefficient is considered to be varying between(10-150)W/m2.K with air as the cooling medium. The temperature distribution of reactor surface is estimated by using finite difference method and the results are compared with the ANSYS9 finite element code results.

The temperature profiles of radiation show different aspects and it is found that the radiation term has not effect on temperature profile, so the radiation can be ignored in the case of forced convection system. While it has a significant effect in the case of free convection system. On the other hand, the validity of thermal results for free and forced convection and radiation effect is good agreement between the results of the numerical solution and ANSYS9 results.

Keywords / Heat Exchanger, Fins, Radiation , Free and Forced convection,ANSYS9.

الخلاصة

إنّ هدفَ هذه الدراسةِ هو تُحليّلَ انتقال الحرارة في الزعانفَ ثلاثية المقطع للمبادلات الحرارية وبيان تأثيراتِ الحملِ والإشعاعِ في نموذجِ انتقال الحرارةِ على مفاعل تحفيزي. أن الحرارة تنتقل بالتوصيل على طول الزعنفة و تشتت الحرارة من السطح بالحمل و الاشعاع.  تم أيجاد توزيع درجات حرارة  على سطحِ المفاعلِ َ باستعمال طريقةِ الفروق المحدودةِ , وتمت الدراسة على جدار واحد للمفاعل نتيجة للتشابه. تم ايضا استخدام برنامج (9ANSYS)  لتصحيح النتائج وإيجاد توزيع درجات الحرارة على الجدار المفاعل المزعنف. ان معاملات انتقال الحرارة المستخدمة في هذا البحث تتراوح بينW/m2.K  ( 10 . (150 - أن مخططات درجة الحرارة تبين ان الإشعاعِ له سماتَ مختلفةَ حيث بينت النتائج عدم تأثير الإشعاعَْ على مخططات درجة الحرارة في نظامِ الحملِ القسري بينما لا يمكن إهماله في حالة الحمل الحر. من الناحية الأخرى، تم التأكد من صحة  النتائج  الحرارية لحالتي الحمل الحر و القسري و تأثير الشعاع حيث كان هناك توافق جيد بين الحل العددي والحل باستخدام برنامج  (9ANSYS) .

2-Nomenclature

| Symbol | Description | Unit |
| --- | --- | --- |
| A | Area | m2 |
| Cp | Specific heat capacity | J/Kg K |
| F | Shape factor | - |
| l | Latent heat capacity | W/m K |
| k | Thermal conductivity | W/m K |
| L | Length | m |
| M | Sub- region number | - |
| q' | Heat flux, | W/m3 |
| q''' | Heat generation rate per volume | W/m3 |
| S | Fin gap | m |
| Tc | Temperature of casting surface | K |
| Tm | Temperature of mold (chill) surface | K |
| T | Temperature | K |
| W | Width of base | m |
| x | distance | m |

Greek symbols

| r | Density of material | Kg/m3 |
| --- | --- | --- |
| σ | Stefan- Boltizman constant | - |
| Є | Emissivity of the surface. | - |
| Dx | Distance interval | m |

Subscript

| a | ambient |
| --- | --- |
| b | base |
| f | fin |
| i,j | node points |
| r | radiation |
| s | surface |

3-Introduction

Heat exchangers are commonly used in many fields of industry, which are composed of finned surfaces for dissipation of heat by convection and radiation. The calculation of heat transfer of a cooling fin in heat exchanger system is the good practical application of heat transfer. Such fins are used to increase the cooling area of system available for heat transfer between metal walls and conducting fluid such as gases and liquids by (Bird et al. 2000). In a chemical process, the reactor at hot temperature is cooled using cooling fins. The coolant is the surrounding air. Heat transfer in heat exchanger is dominated by convection from the surfaces, although the conduction within the fin may also influence on the performance. A convenient method to treat convection cooling is to use heat transfer coefficients, h, by (Welty et al.1997).

A system is catalytic reactor with heat exchanging fins (Fig.1), which is a monolithic bed reactor for auto thermal reaction (ATR). As the reactant gas is introduced into the monolithic ATR reactor, the reaction occurs along the monolith, but the temperature in the reactor is not uniform. The temperature of inlet end rises rapidly, and then gradually decreases. The wall thickness and the length of the heat exchanger are 0.25 m and 8m, respectively. The dimensions of fins are 0.4m*1m. During operation, the temperature inside the wall is maintained. The heat is conducted within the fins and then transferred to the surrounding air. As the air is heated, buoyancy effects cause heat to transport upward by heated air which rises (free convection) or heat sweep to right by forced stream of air (forced convection ) by (Saini et al.,2000).

The temperature of reactor depends on the flow rate and composition of reactant like above diagram. In present work, the temperature at the inlet end of catalytic processor is 823K, and it is increased rapidly to 928K and cooled down to 773K. In the present study, the effect of the convection and radiation has been studied by using finite difference method and ANSYS9 finite element with triangular elements. Typical values of h are given in Table (1) by (Yunus A. Çengel,2007).

Table (1):Typical values of convection heat transfer coefficient

| Type of convection | h, W/m2K |
| --- | --- |
| Free convection of gases | 2-25 |
| Free convection of liquids | 10-1000 |
| Forced convection of gases | 25-250 |
| Forced convection of liquids | 50-20000 |

The ranges of heat transfer coefficient on the surface of the wall heat exchanger fins are varied from 10 to 150 W/m2K. The value for the heat transfer coefficient at the side of reactor is approximated with a 10 W/m2K for free convection. For forced convection, the heat transfer coefficient is approximated with a 150 W/m2K. In this system, the heat transfer coefficient is defined as a constant for a convenience of calculation. Typical material properties of  Steel AISI 4340 are tabulated in Table(2)

Table (2):Material Properties

| Material | k[W/m .K] | [kg/m3] | [J/kg.K] |
| --- | --- | --- | --- |
| Steel AISI 4340 | 44.5 | 7850 | 460 |

4.  Mathematical model

The analysis is based on the following assumptions:

1.Two dimensional  and steady state heat transfer model,

2.All the physical properties are assumed to be constant,

3.There is a perfect contact between the wall and the extended surfaces,

4. The radiation effect is significant,

5. The fluid is considered incompressible with constant properties,

6. There are no heat sources within the fin itself.

(i)Radiation Heat Transfer at V-channel region

The radiation heat transfer coefficient hr, of heat exchanger fins is similar to convection heat transfer coefficient .Based on Stefan  Boltzman equation ,the coefficient, hr, is defined as follows.

L

W

tb

Fig. 2:Notation of Heat sink dimension

qr=FAsσ(T'4−T'4∞)Ts−T∞(Ts−T∞)..........(1)qr=FAshr (Ts−T∞)............(2)hr=σ(Ts'4−T∞'4)Ts−T∞..............(3)

S

Where, F: Gray body shape factor, As: Area of surface, σ: Stefan-Boltizman constant, Ts,T∞: Surface and ambient temp.(OC), Ts',T∞': Surface area and ambient temp(K), by (S.Lee,1994).

Assuming that all external surface of heat sink is black body and, fin and base temperature distribution is uniform . Body shape factor, F, of  the channel by (Donglyoul shin,2004), is defined as follows:

F=2CnetH(S+2L)...............(4)

1

5

4

6

2

3

Cnet=(Ra+Rb+Rc)(Rc+Rd+Re)−Re2and,

Ra=(1−ε)/εA3

Rb=2(1−ε)/εA1

Rc=1/(A1F13+2A3F35)

Rd=2/(A1F12+2A1F15)

Re=1/A1F13

Ri : is the surface resistance to radiation.

A1,A3: is surface Area of surface1,3

Fij: is shape factor from surface i to j

Fig. 3: Heat sink surface and Facing air

(ii)- Heat flow through the fin.

The fin is finite length and loses heat by convection from its ends with radiation.

qx

qy

b

L

qconv

qx+dx

qy+dy

qr

qconv

qr

a: Thermal Analysis through the fin.

dqin−dqout=0..................(6)dq−dq+dqy−dqy+dy=0...............(7)

From Fourier law gives

dqx+dx=dqx+ddx(qx)dx....................(8)

dqy+dy=dqy+ddx(qy)dy....................(9)

Fig.4: Heat balance in fins

Substitution of Eq.(8,and 9), in Eq.(7), gives

ddxk.AdTdxdx+ddyk.AdTdydy=0...............(10)

d2Tdx2+d2Tdy2=0...............(11)

b: Thermal Analysis at the wall of the fin.

dqin−dqout≡0..................(12)dq−dq+dqy−dqy+dy−dqconv−dqrad≡0...............(13)

The convection heat transfer rate may be expressed as

dqconv = h P.dx. (T -T∞)                 ……………..(14)

Substitution of Eq.(2,8,9and14), in Eq.(13), gives

ddxk.AdTdxdx+ddyk.AdTdydy−2h.p..T−T∞dx−FAsσT4−T∞4=0...............(15)

This equation can be rearranged as

d2Tdx2+d2Tdy2−m1.T−T∞−m2T4−T∞4=0...............(16)

where m1=2hpkA , m2=FASσkA................(17)

(iii)- Finite difference formulation

Finite difference methods are use to solve Eq.11, and Eq.16. Equation11 represent temperature distribution inside the fin, on the other hand, the equation16 represent temperature distribution along the wall of the fin ( the wall is irregular), Now construct mesh along fin as shown in  fig.5 .In the finite difference analysis of two-dimension conduction of element ,The center of finite difference are used for grid as shown in Fig.6 ,the eq.11  become,

j

i

Ti-1,j

Ti,j

Ti+1,j

Ti,j+1

i

j

Ti,j+1

Ti-1,j

Ti,j

Ti+1,j

Ti,j-1

Ti,j-1

Ti,j−1+Ti−1,j+Ti,j+1+Ti+1,j−4Ti,j=0................(18)

:

Fig.5: Temperature distribution in Fins

Ti,j+1

Ti-1,j

Ti,j

Ti+1,j

Fig.6 :Temperature distribution in irregular regions

h

Ti,j-1

This equation used for all nodes in the fin respect the nodes near the wall analysis as irregular regions and used equation (16) by (Eslinger and Chung,1979). The temperature distribution in irregular regions in the Fig.6 are used for all nodes near the wall and the dimension different for  these nodes.

∂2T∂x2=Ti+1,j−Ti,jθ3h−Ti,j−Ti−1,jθ1h12(θ1+θ3)h...........(19)

∂2T∂y2=Ti,j−1−Ti,jθ4h−Ti,j−Ti,j+1θ2h12(θ2+θ4)h...........(20)

Now, substitutes Eq. 19,and Eq 20 in Eq.16 ,get

2h2Ti-1,jθ1(θ1+θ3)+Ti,j+1θ2(θ2+θ4)+Ti+1,jθ3(θ1+θ3)+Ti,j−1θ4(θ2+θ4)−(1θ1θ3+1θ2θ4)Ti,j-m1(Ti,j-T∞)-m2(Ti,j4
                  -T∞)=0...........(21)

Equation 18 and 21 represent temperature distribution along fin after construct computer program to solve these equation and depend on boundary condition also we needed Newton- Rafson method to solve Eq 21 because the equation is non-linear and Gaues Elimination method to solve Eq 18.

(Iv)- The boundary conditions  :

From the symmetrical of the system, the boundary conditions are:

a) Base surface:

Aty=0and0x≤W2 T=1.866x4−29.498x3+132.27x2−157.957x+823 ..............(22)Aty=0 and W2x≤W T=1.553x4−29.498x3+182.801x2−417.72x+1157.178. ...........(23)

b) Fins tip:

Aty=L−K.A.dTdxx=L=hP(TL−T∞).........(24)

5. Results Verification:

In order to verify our results obtained using ANSYS9 based finite element. In order to be consistent with what we have for ANSYS9 based finite element model, half size of the elements in z direction is used adjacent to the symmetry line. The half model shown in Figure 7 is generated using ANSYS9. The wall is designed to be sufficiently thin so that the temperature variations across the thickness of the wall are important.

Fig.7:Meshing of  the half model

6. Parametric Study.

1-The effect of convection only.

Figure8 shows the temperature profile on fin number four according to four typical values of heat transfer coefficients (h), namely 10,50,100 and 150 W/m2K by considering emissivity (ε=0.3). Temperature drops linearly toward the end of fin around (938 to 510)K when the convection term is free, but as the heat coefficient is getting increased (forced convection), the temperature drops sharply around (938 to ≈ 300)K. When the convection heat transfer coefficient is large, the convection resistance becomes small(R=1h.As).

Fig. 8. The effect of convection on  fin number four.

Fin Length(m)

Fig. 8. The effect of convection on  fin number four.

2-The effect of convection and radiation in heat dissipation.

In this case, the surface heat dissipation from the geometry is occurring both by convection and radiation. Figure 9 shows The effect of convection and radiation on fin number four in the heat exchanger (maximum heat dissipation from this fin). The free convection heat transfer coefficients are typically very low compared to those for forced convection so, the free convection cooling is limited to heat dissipation. For free convection, at the tip of fin, the temperature difference is θ≈34K. While, for forced convection, the temperature difference is just θ≈0.35K, which tells the radiation is usually disregarded in forced convection (Fig.10).

Temperature (K)

Fin Length(m)

Figure (9): The effect of convection  and radiation on  fin number four.

Fig.10. Comparison of Temperature distribution profiles at both free and forced convection, and radiation or without radiation

7. Results

From the result of free convection (Fig.11a), the heat is dispersed well throughout the reactor wall and the temperature range is from 436 to 470K.  All cross-section temperature profiles are converged into around 436 K at the end of the fins. The comparison among the temperature profile of surface and insides shows the effect of convection. Two temperature profiles of insides are similar (the temperature difference between insides of the fin is around 54K), but for forced convection (Fig.11b), the temperature at the surface is lower than those of inside (temperature difference between surface and inside is around 0.7K). This means the forced convection remove heat much higher rates than the free convection for all fins.

(a) Free Convection( h=10W/m2.K)

(b) Forced  Convection(h=150 W/m2.K).

Fig.(11a&b): Temperature profile through the all fins in heat exchanger.

Figure 12 is the results from numerical calculation and ANSYS9. The calculation of Free convection by numerical solution shows similar result to ANSYS9 result. However, in Forced convection, the numerical calculation is lower than that of ANSYS9. Even though the temperature profile of Forced convection has differences between ANSYS9 and Numerical calculation, the temperatures at the end of the fin are same.

Table(3): The Minimum Temperature in the system using ANSYS9, results from Appendix(A).

| Convection Type | Tmin(K) |
| --- | --- |
| Forced convection without radiation | 298.584K |
| Forced convection with radiation | 298.24 K |
| Free convection without radiation | 470.259 K |
| Free convection with radiation | 436.027 K |

Fig. 12. Comparison between forced convection and free convection

8. Conclusions

1-The radiation heat transfer in forced convection cooled finned surfaces is usually disregarded for two reasons. First, forced convection heat transfer is usually much larger than that due to radiation, and the consideration of radiation causes no significant change in the results. Second, the heat exchanger fin convection cooled systems are mounted so close to each other that a component is almost entirely surrounded by other components at about the same high temperature. That is, the fins have hardly any direct view of a cooler surface. This results in little or no radiation heat transfer from the fins. The fins near the edges of heat exchangers with a large view of a cooler surface may benefit somewhat from the additional cooling by radiation.

2- The radiation effect is most significant when free convection cooled finned surfaces due to convection heat transfer coefficient is small (thus free convection cooling is limited).

9. References

Bird, Stewart, lightfoot, (2000)“Transport Phenomena”, John Wiley & Sons, p. 265-309.

Donglyoul shin,(2004)," Thermal Design and Evaluation methods for Heat Sink" E - CIM  Team,

Corporate Technical Operations

Elsworth,(1999) “Course Notes and Resources : Mathematical Modeling of Energy and Geo-Environmental Systems”

Saini, Manish,(2000) “Modeling and optimization of air cooled plane fin aluminum heat sinks used in computer cooling”, MSc  Thesis , Pennsylvania State University.

S. Lee,(1994), "Optimum Design and Selection of Heat Sinks", IEEE SEMITHERM Symposium.

Welty, Wicks, Wilson,(1997) “Fundamentals of Momentum, Heat, and Mass Transfer, 3rd edition”, John Wiley & Sons, p.252-295.

Yunus A. Çengel(2007)'' Heat Transfer  A practical Approach" Mc GRAW-HILL BOOK COMPANY, p.26

10.Appendix

a)h=10W/m2.K

b)h=100 W/m2.K

c)h=50 W/m2.K

d)h=150 W/m2.K

Fig.(A):Temperature distribution profiles for

ANSYS solution (The effect of convection )

a) h=10 W/m2.K

b) h=100 W/m2.K

h10

h100

d) h=150 W/m2.K

c) h=50 W/m2.K

Fig.(B):Temperature distribution profiles for

ANSYS solution (The effect of convection and Radiation)