Statistical Analysis and Computer Program for Estimating Health Impacts from Consumption of Contaminated Drinking Water

Sundus  Saleh Nehaba			Alaa H.  Wadi Al-Fatlawi

College of Engineering/ Babylon University

Abstract

This study was conducted to assess the  impact of water pollution on human health. In this study the statistical models which are described the relations between the water quality parameters and number of  infections of cholera, diarrhea, typhoid, and hepatitis diseases were established. Data used in the statistical models consist of monthly means of water quality parameters and the monthly mean of number of  infections for each disease. Also a Visual Basic 6.0 language  program was constructed to evaluate the risk in water supply system.

الخلاصة

أجريت هذه الدراسة لتوضيح تأثير المياه الملوثة على صحة الإنسان. في هذه الدراسة وضعت الموديلات الإحصائية التي تصف العلاقة بين خصائص المياه وعدد الإصابات لكل من مرض الكوليرا، الإسهال، التيفوئيد، والتهاب الكبد الفيروسي وان البيانات المستعملة في الموديلات الإحصائية شملت المعدلات الشهرية لكل من خصائص المياه وعدد الإصابات لكل مرض. وأيضا تم إنشاء برنامج حاسوبي بلغة فيجوال بيسك (6) كأسلوب مرن وبسيط لتقييم الأخطار في نظام تجهيز المياه.

- Introduction

The failure to provide safe drinking water to all people perhaps the greatest development failure in recent century. The most egregious of this failure is the high rate of mortality among human spatially child from water-related disease. As well as water is consider as an important way of human exposure to contaminant.

Drinking water supplies continue to be a major source of human disease and death globally because many of them remain unsafe and vulnerable. Greater efforts are needed to address the key issues and questions which influence the provision of safe drinking water. Efforts are needed to re-evaluate and set new and better priorities for drinking water research and practice (Sobsey, 2006).

Contamination of drinking-water is a significant concern for public health throughout the world. Microbial hazards make the largest contribution to waterborne disease in developed and developing countries. Nevertheless, chemicals in water supplies can cause serious health problems. The drinking water must be free from impurities and bacteria (Thompson  et al., 2007).

- Statistical analysis

The statistical model for each disease was generated  to raw, treated, and distributed tap water. The regression analysis was done by using  SPSS program version 17 .Stepwise multiple linear regression models were used for each model. multiple regression procedures will estimate a linear equation of the form (Draper and Smith, 1966):

……..(1)

Where:

y = dependent variables.

x1, x2, …, xn = the independent variables.

b1, b2, b3, …bn = are model coefficients,

G = model constant term.

Standard linear form of  Equ. (1) was found to be the most suitable for all regression models. The best fitting of data gives the best model were selected on the basis of the higher coefficient of determination value (R2) and smaller value of the standard error of the estimate.

- Data collection and analysis

Data concerning raw and treated water quality parameters  were collected from (Babylon Water Directorate) and (Babylon Environmental Directorate) for the period extened from January 2008 to  December 2009. While the data concerning the monthly means of infection cases of cholera, diarrhea, typhoid, and hepatitis disease were collected from ( Babylon Health Directorate) for the period extent from January 2008 to  December 2009.

In the present study, the number of infections for each disease (cholera, diarrhea, typhoid, and  hepatitis) were  taken as dependent variable (y), and the  parameters of water  quality of raw, treated, and distributed tap water were taken as independent variables. Table (1) shows these variables.

Table (1): Description of independent variables

| Independent variable | Description |
| --- | --- |
| X1 | Turbidity (NTU) |
| X2 | Hydrogen ion concentration (pH) |
| X3 | Nitrate (NO3 , mg/L) |
| X4 | Total dissolved solids, (TDS, mg/L) |
| X5 | Chloride, (CL-, mg/L) |
| X6 | Phosphate (PO4 ,mg/L) |
| X7 | Total hardness (T.H, mg/L) |
| X8 | Sulphate (SO4, mg/L) |
| X9 | Potassium (K, mg/L) |
| X10 | Temperature (T,Co) |
| X11 | Alkalinity (Alk., mg/L ) |
| X12 | Chlorine(free) (Cl , mg/L) |
| X13 | E.coli (cell/L) |

öAs example the results of statistical analysis can be seen in table (2). This table shows the summary of each model for treated, and distributed tap water in Al-Hashimeya city.

|  | Period Date | Depended variable(y) | model | Stand. Err. | R2 |
| --- | --- | --- | --- | --- | --- |
| Treated water | 2008-2009 | Number of infection for cholera disease | 0.94*x1+4.25*x2-0.31*x3+2.97*x4+20.95*x6+0.05*x7+0.05*x8+0.24*x10-69.96 | 4.820 | 0.788 |
|  |  | Number of infection for diarrhea disease | 8.14*x1- 40.98*x2+68.47*x3+0.08*x4 +0.57*x6- 0.34*x7+434.52*x8+1.43* x10- 14.01*x12+554.11 | 17.66 | 0.929 |
|  |  | Number of infection for typhoid disease | 2.34*x1+6.64 *x2 +0.01*x4+ 70.34*x6- 0.03*x8 -4.1*x9 +0.46X10– 1.6*x12+ 168.01 | 6.510 | 0.734 |
|  |  | Number of infection for hepatitis disease | 0.08*x1 +1.92*x3+0.001*x4 +18.79*x6+0.03*x7-0.01*x8+0.07*x10-0.64*x12+9.19 | 1.318 | 0.777 |
| Distributed water | 2008-2009 | Number of infection for diarrhea disease | 6.75*x1 – 39.61*x2 – 15.96*x3 + 0.65*x8 – 17.12*x12+ 1.75*x13+628.9 | 34.438 | 0.711 |
|  |  | Number of infection for typhoid disease | -0.17*x1 + 1.071*x2 – 1.83*x3 + 0.05*x8 + 0.27*x10+ 1.74*x12+ 0.20*x13+28.48 | 2.153 | 0.767 |
|  |  | Number of infection for hepatitis disease | - 0.02*x1+ 0.68*x3 +0.14*x10 + 1.68*x12+0.31*x13+9.311 | 3.725 | 0.700 |

Table (2): Statistical analysis results for treated and distributed water in Al-Hashimeya city .

- Computer Program, Description and Application

A computer program was written by using Visual Basic 6.0 language to assess the risk in water supply system. This program had been selected to be easy to use and localization to any area or future applications. The program was divided into two parts : first named ″risk assessment of contaminates in water″, and the second named ″stochastic approach″.

Fig. (1) shows the flow chart of the program and the steps of the program are as follows:

First : Risk Assessment of Contaminants in Water

This part of program calculates the intake, hazard index, and risk for concentration of contaminants in water . The procedures of this part of  program are as follows:

- From the front page of the program in Fig.(2) click (risk assessment of contaminants in water ), botton.

- From  Fig. (3) select the type of risk (chemical or microbial risk assessment)  as follow :

- Chemical risk assessment

- Input the concentration of chemical in water.

- Input parameters required for calculated the intake and hazard index or risk.

- From combo box in Fig. (4)  select the type of risk required as follow:

- HI selected for noncarcinogenic chemicals in water

- Risk  selected for carcinogenic chemicals in water

- Select calculation botton to show the results. These results include intake, noncarcinogenic (HI) or carcinogenic  Risk, and comparison of these results with EPA limitations to show if it’s acceptable or not.

- Microbial risk assessment

- Input the concentration of organism  in water.

- Input parameters required for calculated the probability of infection .

- From combo box in Fig. (5)  select the type of probability of infection  as follow:

- PI1 to calculate risk of E.coli in drinking water.

- PI2 to calculate risk for diarrhea caused by E.coli in drinking water.

- Select calculation botton to show the results. and comparison these results with EPA limitations to show if it’s acceptable or not.

Table (3) show the equations using for calculation the chemical and microbial risk assessment :

Table(3): The equations using for calculation the chemical and microbial risk assessment

| Chemical risk assessment (Davis, and Masten 2004) | Microbial risk assessment (Fewtrell and Bartram, 2001 & Navier et al., 2006) |
| --- | --- |
| ……………..(2) ………………(3) Risk = CDI × SF …………..(4) Where I= intake by ingestion (mg / kg.day) C= chemical concentration in water (mg/L) IR= ingestion rate (L/day) EF= exposure frequency (day/year) ED= exposure duration (years) BW= body weight (kg) AT= average time =ED × 356 days/year . HI = hazard index (dimensionless) I= intake (mg/kg.day) RfD= reference dose (mg/kg.day) Risk = the probability of carcinogenic risk (dimensionless) CDI= chronic daily intake (mg/kg.day) SF= carcinogenic slop factor (kg.day/mg) | …..(5) PI1 = (PI (infection/day)) ×365 …..(6) P I2= PI (infection/year) × P (ill/infection) …..(7) Where N50= median infectious dose =8.6×107 α= slope parameter = 0.1778 d=V×C d= Ingestion dose V=Consumption of drinking water (2 L/day). C= Exposure by drinking water, organisms per liter . |

Second: Stochastic Approach

This part of program using the outputs data resulted from statistical analysis which conduct on number of infections for each disease. The procedures of this part of  program are as follows:

- From the front page of the program in Fig.(2) click (stochastic approach), botton.

- From comb box in Fig.(6) select the type of water and location.

- Input the parameters included in the regression model.

- Compare these parameters with Iraqi, WHO, and  EPA standards by click comparison  botton.

- From combo box in Fig.(9) selected the type of disease, to calculate number of infections according to the regression model.

- Click map botton to show the number of infections and its location on the map as shown in Fig.(10).

Start

Select the type of water           (raw, treated, tap) and location

Input data                                                 x1,  x2,  x3, x4, x5,  x6, x7, x8 , x9, x10, x11, x12 & x13

X1

Compare with Iraqi, WHO,EPA standards

Output data (y)    number of infections

Show output data on map

END

Stochastic Approach

Microbial risk assessment

Chemical risk assessment

Input parameters required C, IR, EF, BW, AT, RfD, SF

Select  risk type HI or Risk

Calculation

END

Input parameters required C, V, α, N50

Select  risk type PI1 or PI2

Calculation

END

Select type of risk

Fig.(1): Flow chart of computer program

Fig.(2): Front page of program

Fig.(3): Page of type of risk to calculate risk for contaminants in water

Fig. (4): Page of calculating intake and risk for lead in raw water.

Fig.(5): Page of calculating risk for E.coli in treated water.

Fig.(6): Page of stochastic approach to selected type and location of water

Fig.(7): Page of water quality parameters of Shatt Al-Hilla in Al-Hashimeya station.

Fig.(7): Page of input data.

Fig.(9): Page to calculate number of infections.

Fig.(8) page of input data

Fig.(10):Page to show the number of infections on map

5. Conclusions

According to this study, the following conclusions can be drawn up.

- The statistical results showed  a good positive  correlation between number of infection of cholera, diarrhea, typhoid and  hepatitis  disease with quality parameters of raw, treated, and distributed water.

- The computer program can be considered as a guide to the application of rapid technique to assess and monitoring risk.

- The results of the study help in enhancing awareness of health hazards of contaminated drinking water  among consumers as well as in drawing attention of health regulatory authorities.

6. References

Davis, M. L. and  Masten, S.J., (2004), ″principles of environmental of engineering and science″, Michigan state university-East Lansing , book .

Draper, N.R., and Smith, H., (1966), ″Applied Regression Analysis″, John Wiley and Sons Book Companies, Inc., U.S.A.

Fewtrell, L. and Bartram, J., (2001) ,″water Quality: Guidelines, Standards and Health″ Published by IWA Publishing, London, UK. ISBN: 1 900222 28 0

Navier, D.,  Laroche, L. and Hartemann, P. (2006),″Waterborne microbial risk assessment″, Grenoble University Hospital.

Sobsey, M.D., (2006) , ″ Drinking water and health research: a look to the future in the United States and globally″, Journal of Water and Health, 04.Suppl, IWA Publishing , University of North Carolina.

Thompson, T., et al.,  (2007), ″Chemical safety of drinking water : Assessing priorities for risk management ″, Geneva , Switzerland. ISBN 978 92 4 154676 8.