Cost- Benefit Estimation For Alternative Renewable Biofuel From Algae in Comparison with Traditional fuel Source

Eman Mohammed Abdullah

Chairwoman of accounting Department/Babylon technical institute of Babylon

Abstract

In the face of the problem of depletion of natural resources, and preservation of  environmental and ecological balance. the measuring tools of environmental and social  accounting, stand  in front of  challenge in finding method to measure the change in cost of  production that used new methods in produce a new resource of energy,  which is friendly to the environment and eliminated the pollution. One of this  alternative renewable resources is Algal. as promising sources of clean energy products and a CO2 capturing from air. but the requirements of the complex processes  used into convert the Algae to biofuels, create cost structure  not competitive with the traditional fuels. So, in this paper it will built a theoretical  cost structure ,and aggregate  cost elements by using accounting terminology with interaction with terminology chemical engineering, biology and economics field, to calculate cost by  equations, and design software program (MS-DOS program) to evaluate the production cost, and the potential economic viability of algae in producing fuel instead of producing it from conventional sources.

Key words: cost-benefit model, operation costs, capital costs, productivity, Algal-Biofuel

الخلاصة

لمواجهة مشكلة استنزاف الموارد الطبيعية وللحفاظ على التوازن البيئي والايكولوجي , تقف ادوات قياس المحاسبة البيئية والاجتماعية امام تحدي لايجاد طريقة لقياس التغير في تكلفة الانتاج التي تستخدم اساليب جديده في انتاج موارد جديدة للطاقة   تكون صديقة للبيئة وتساهم في القضاء على التلوث. واحدى هذه المصادر المتجددة هي الطحالب. التي عدت مصدرا واعدا لانتاج الطاقة النظيفة ولامتصاص ثاني اوكسيد الكاربون من الجو. لكن متطلبات العمليات المعقدة المستخدمة في تحويل الطحالب الى وقود حيوي خلق هيكل تكاليف غير فادر على المنافسة مع الوقود التقليدي . لذا سنعمل في هذا البحث على البناء النظري لهيكل التكاليف المطلوب, وتجميع عناصر الكلفة الاجمالية لهذا البناء النظري باستخدام المصطلحات المحاسبية مع التفاعل مع مصطلحات حقل الهندسة الكيمياوية والبايولوجية والاقتصادية لحساب التكلفة وفق معادلات وبرنامج               وتقييم تكاليف الانتاج ومعرفة الجدوى الاقتصادية المحتملة من استخدام هذا البديل في انتاج الوقود بدلا من انتاجة من المصادر التقليدية

| Notation used No | parameter | symbol | Unit |
| --- | --- | --- | --- |
| 1 | Total capital cost | TCAP.C | $ |
| 2 | Arial productivity | AR.P | g/l.d |
| 3 | Annual productivity | AN.P | Ton/year |
| 4 | Annual productivity | AN.P1 | kg/year |
| 5 | Annual (N) cost | AN.C | $/y |
| 6 | Annual (P) cost | AN.P.C | $/y |
| 7 | Annual CO2 cost | AN.CO.C | $/y |
| 8 | Annual fresh water cost | AN.FW.C | $/y |
| 9 | Annual labour cost | AN.L.C | $/y |
| 10 | Annual power cost | AN.PO.C | $/y |
| 11 | Total cost | TOTAL.C | $ |
| 12 | Operation cost | OperationC | $ |
| 13 | X=unit cost before harvesting | X | $/kg |
| 14 | Y=unit cost after harvesting | Y | $/kg |
| 15 | Z=unit cost before+ after harvesting | Z | $/kg |
| 16 | Number of hectares | No. ha | - |
| 17 | Biodiesel productivity | BP | gallon/ye |
| 18 | Biodiesel productivity | BP1 | barrel/ye |
| 19 | Gross annual revenue | GA.R | $/ y |
| 20 | Benefit | benefit | $/ y |
| 21 | Contribution of nitride to total operation cost | J1 | % |
| 22 | Contribution of phosphors to total operation cost | J2 | % |
| 23 | Contribution of CO2 to total operation cost | J3 | % |
| 24 | Contribution of labour to total operation cost | J4 | % |
| 25 | Contribution of power to total operation cost | J5 | % |
| 26 | Contribution of water to total operation cost | J6 | % |
| 27 | Contribution of indirect costs to total operation cost | J7 | % |
| 28 | Total pond volume | TPV | Ha |
| 29 | Used nitride from culture media | NUC | g/l |

| 29 | Total Pond Area | TPA | ha |
| --- | --- | --- | --- |
| 30 | Cost of Site preparation | C.SIP | $/ha |
| 31 | Cost of Culture system | C.CLS | $/ha |
| 32 | Engineering fee | ENGF | $/ha |
| 33 | Contingency | COG | $/ha |
| 34 | Cost of Land | C. Land | $/ha |
| 35 | Total growth days | TGD | Day |
| 36 | Proportion of down time | PDW | % |
| 37 | Pr-+oportion of pond harvesting | PPH | % |
| 38 | Cost of nitride | CNI | $/kg |
| 39 | Cost of phosphors | CPHt | $/kg |
| 40 | Harvesting efficiency | HE | % |
| 41 | Pond depth | PD | M |
| 42 | Pond length | PL | M |
| 43 | Pond width | PW | M |
| 44 | No of ponds | NOP | - |
| 45 | Used nitride from culture media | UNCM | g/l |
| 46 | Proportion of medium recycled | PMR | % |
| 47 | Algae doubling time | TD | Day |
| 48 | NaNo3 concentration in medium | NCM | g/L |
| 49 | Used phosphors from culture media | UPCM | g/L |
| 50 | NaH2PO4.1 H2O concentration | PCM | g/L |
| 51 | Volumetric productivity | VP | g/l.d |
| 52 | Co2 cost | CO2.C | $/L |
| 53 | Co2 required for different ph | Co2Nph | L/d |
| 54 | Fresh water cost | FW.C | $/m3 |
| 55 | Average days of evaporation | ADE | D |
| 56 | Rate of evaporation | RE | M |
| 57 | Labour cost supervisor | LCS | $/ha.y |
| 58 | Labour cost senior technision | LCST | $/ha.y |
| 59 | Labour cost technision-day term | LCTD | $/ha.y |
| 60 | Labour cost technision-shift term | LCTS | $/ha.y |
| 61 | Power cost | PO.C | $/kw.hr |
| 62 | Power usage | POU | Kw/ha.d |
| 63 | Harvesting system cost annually | AHC | $ |
| 64 | Lipid yield | L.Y | % |
| 65 | Price of Gallon | Price .G | $ |

1. Introduction

The world has realized that the basic cause of the energy crisis is not just scarcity but also the lack of knowledge and the limitation of nature, it is necessary for the world to engage in research to push for alternative fuels and to develop new sources of energy which are renewable and inexhaustible .The problem of Petroleum shortages and the climate implications proven reserves to have driven research and business ventures into algae-based fuels (IEAWEO, 2007). Although efforts to produce renewable energy on an industrial scale have been started in many alternative renewable energy sources like solar, wind, corn, and so on, but produce oil from algae, is one of the most promising sources of alternative energy. According to the historical generation of biofuels industry, revolutions happened in biofuels energy industry characterizing algal biofuel production as a third revolution (IEA Bioenergy, 2008), (http://www.altprofits.comref/report/biofuels) as shown in fig 1.

Figure.1 Structure of generation biofuels industry revolutions

Because of the viability of the 1st and 2nd generation biofuels production is however questionable and conflict with either food supply or the fact that may not be enough land to grow the necessary amount of feedstock’s. So as an alternative to corn, sugar cane, many believes that algae is set to eclipse all other biofuel feedstock’s as the cheapest, easiest, and most environmentally friendly way to produce liquid fuel. Because of this revolution the promise of sustainable energy production from algae has generated tremendous interest in recent years (Antoni.D.et al, 2007) (Srivastava .A, 2000). It considered as a viable alternative biofuel feedstock. This does not came easily, it need more subject and huge systems in order to cultivate and convert algae into a biofuel. For this the subject need to offer briefly in order to give it reserves.

Algae can be referred to as plant- like organisms that are usually photosynthetic and aquatic, but do not have true roots, stems, leaves, vascular tissue and have simple reproductive structures. They are distributed worldwide in the sea, in freshwater and in wastewater, most are microscopic, but some are quite large, e.g. some marine seaweeds that can exceed 50 m in length (http:WWW.oilgae.com/algae/algae.htm, introduction to algae &types of algae). Algae are the most diverse organisms in the world (www, algaebase.org), it divided into two categories distinction between them as list in Table 1.

Table 1. Distinction between macro and micro algae

| Macro algae Microalgae |  |
| --- | --- |
| 1- Commonly called “seaweeds” 2-Properly called “sea plants” 3-Big enough to tie on ropes 4-Many can be chopped down to “mini”size Like: 1-Green algae 2-Brown algae (Kelp) 3-Red algae | 1-Too small to see with naked eye 2-Best grown in slurry systems 3-Some grown in open system 4-Must be enclosed for pure cultures Like: a-Blue green algae (usually benthic) b-Diatoms (major phytoplankton group, can be benthic) c-Din flagellates (major phytoplankton group) d-Others, including raphidophytes |

Microalgae have many different species with widely varying compositions and live as single cells or colonies without any specialization. Although this makes their cultivation easier and more controllable, their small size makes subsequent harvesting more complicated. Macroalgae are less versatile, there are far fewer options of species to cultivate and there is only one main viable technology for producing renewable energy: anaerobic digestion to produce biogas. Both groups will be considered, but there is more research, Practical experience, more fuel options from microalgae, for this it take a bigger share in most research (GBEP, 2009). Biologists have categorized, Microalgae in a variety of classes, mainly distinguished by their pigmentation, lifecycle and basic cellular structure, but the most four important are diatoms (Bacillariophyceae) , green algae (Chlorophyceae) ,blue-green algae (Cyanophyceae), golden algae (Chrysophyceae)

(NREL, 1998).There are more than (30000) to (100000) kind of strain of algae, each kind includes many species (Nichols, J), but Researchs focused on Microalgae for mass-production of oil, the preference toward microalgae is due to its less complex structure, fast growth rate, and high oil content (for some species),and some types of algae comprise more than 50 percent oil, as shown in Table 3. (Chisti, Y.2007).

Table3. Oil Content of some Microalgae

| Microalgae | Oil Content (%dry wt) |
| --- | --- |
| Botryococcus braunii Chlorella sp Crypthecodinum cohnii Cylindrotheca sp Dunaliella primolecta Isochrysis sp Monallanthus salina Nannochloris sp Nannochloropsis sp Neochloris oleoabundans Nitzschia sp Phaeodactylum tricornutum Schizochytrium sp Tetraselmis sueica | 25-75 28-32 20 16-37 23 25-33 >20 20-35 31-68 35-54 45-47 20-30 50-77 15-23 |

Above all, the average acre of algae grown today for pharmaceutical industries can produce 500 gallons (19000 liters) of biodiesel each year in comparison to an average acre of corn produces 429 gallons (1600 liters) of ethanol per year, and an acre of soybeans yield just 70 gallons (265 liters) of biodiesel per year. From previous the essence issues is economics and cost - benefit  oil production from Algae.

2- Background of the research:

All R&D since century ago, from first breakthrough to discover algae, when pond scum (Anabaena cylindrical, a cyan bacterium) collected from a Massachusetts reservoir was found to produce almost pure hydrogen gas (Jackson and Ellms, 1896), and subsequent time, like offered in 1948, when Paul Cook engaged in some of the first research on algae mass culture and cultivation with Stanford Research Institute, and In 1950s, at USA when algae biomass production for wastewater treatment and conversion to methane (Oswald & Golueke, 1960).Then In 1953,were microalgae biofuels mentioned in conjunction with an algae pilot plant operated on a rooftop at MIT (Burlew, 1953).This early basic research and other’s laid the foundations for the applied research in Algae biofuel production, but in 1970 this was strongly initiated after the energy crisis , at 1978 algae were first explored as alternative fuel in USA, the aquatic species program run by the national renewable energy laboratory, researched high oil-output algae for biofuel, after testing more than 3000 types of algae, but  the subject studied from the view of chemical, engineering, biological...and ect, even in few cost studies in the early and in latest literature, the calculated cost of product came from the view of economics field, and they are differ in determining the cost of gallon as shown in Table 4, and Figure 2.

| Author | Year | Product | Productivity (t/acre/y) | Total costs $ |
| --- | --- | --- | --- | --- |
| Fisher | 1935 | Food | 35 | 49.5 |
| Oswald&Golueke | 1958 | Electricity | 30 | 5.85 |
| Benemann et al | 1976 | Methane | 23 | 7.4 |
| Benemann et al | 1977 | Methane | 20 | 9.4 |
| Dynatech | 1977 | Methane | 30 | 13.7 |
| Tahal | 1978 | Feed | 60 | 184.7 |
| Shelef et al | 1977 | Feed | 60 | 68.8 |
| Kawaguchi | 1977 | Health foods | 25 | 1292 |
| Soeder | 1977 | SCP | 36 | 186.4 |
| Tahal | 1977 | Spirulina | 32.5 | 98 |

Table 4. Summary of microalgae cost analyses published

Figure 2. Production cost of gallon algae oil range in different latest studies

But no study came from the view of accounting ,so in this research ,the subject study f

rom the view of accounting to calculate the cost of barrel of biofuel of algae and evaluating the parameter such as operation cost, capital cost, and productivity that affect on total cost, comparative with benefit that gain from pricing algae oil versus traditional oil price in order to report whether the oil production from algae economically feasible compared to conventional oil.

3.   Algae production systems  ( Input Materials and Methods)

3-1. Input Materials : The inputs materials for production biofuel from algae are simple, all materials needed are the single-celled organisms that only need sunlight, water (fresh or waste) with (nitrate, phosphate), and carbon dioxide Co2 to grow; they can quadruple in biomass in just one day. These inputs and materials according to the accounting classification named initial direct cost includes raw materials like:

• Microalgae that grow rapidly and have high oil yield

•Inexpensive land

• Water

• Light

• Favorable temperature

• Nutrients (N, P, CO2)

• Sustainable culturing approach

• Process by  (photosynthesis) Microalgae convert sunlight, water and carbon dioxide into biomass contains lipid oils and oxygen.

And add to them labour and direct & indirect industrial expenses to be operation cost. Either the cost of land and equipment used in production classified as capital cost

3-2. Production system (Methods) : Producing biodiesel from algae biomass is not a simple technical process. Algae like any good must pass many stages in order to convert it from plant to Oil. Algae had specific condition make it more complex than other goods. One major concern is to determine the important stages in algae production system,( which help us in determining the various components of total production costs) as shown in Table 5 which Summarized these specific stages.

Table 5. algae production

| Cultivations method | Production Stages of Bio-fuel |
| --- | --- |
| Open Pond System |  Flocculation Mechanical Press Mechanical Oil Drum Dryer  Methane Gasification Expulsion |
| Hybrid System |   Liquefaction or Fast Pyrolysis  Mechanical Press Microfiltration Green DieselHydroprocessing |
| modular (indoor)Closed Photobioreactor |  Methanol Fermentation Sonication |
| Heterotrophic Fermentation |   Solvent Extraction  Rotary Dryer  Mechanical Press Centrifugation BiodieselTransesterification |
| Integrated Cultivation System |   Modified Fermentation  Supercritical Fluid Extraction Solar Drying Butanol + Hydrogen |

Sources: (catie R, elt. 2009)

Where cultivation algae are divided into many categories as below;

- Cultivation in pond (Whether close pond or open pond).

- Cultivation in photobioreactor which divided to subsystems in PBR and photobioreactors. And photobioreactors are divided to many categories like;

- Stirred tank.                                                                         e-   Air lift.

- Bubble column.                                                                   f-   Immobilized.

- Flat plate.                                                                            g- Vertical column.

- Tubular reactor (divided to:–Horizontal -Vertical).

- Cultivation in Sewage waste water.

- Marin environmental.

- CO2 capture.

- Desert.

But some, who had active research, and laboratory experiences, had some conversions about algae fuel, this issues title as assumption like: a- The present cost of algae production from open ponds is too high to make fuel production economically viable. b- The photo bioreactors (PBRs) are too expensive. c- The energy inputs into the algae production process are very high. d- Some algae don’t need sunlight, and can produce oil in a fermented process. These issues and the complex processing requirements, create a cost structure that’s not competitive with the traditional fuels .But the largest issue concerning the development of algae into biofuel is the capital cost for a pond or PBR process and this for it self need separate research to measure it.

4.Developing  Cost Equation Model and Calculating the Cost oduction

The more complementary and complex approach for evaluating the cost of a particular biofuel algal produce is to setup the whole process, determine the approximate production costs and assess which factors are the most effected on the final product cost. So depending on the prior studies which addressed key issues as biological, engineering, and economics parameters in development production system to microalgae-fuel like, Productivity (Arial & Annual productivity), Space required, Water losses, O2 inhibition, Process control, Capital/operating cost ....etc (Benemann.J, 2008), we will design assumptions for all parameters. take in consideration uncertainties involved in designing such model, uncompleted, unavailable important information and data about the subject ,because of it under research and developed. Also we use Sensitivity analysis in order to determined which parameters are most likely to have the largest impact on production costs. As we mention previously , this model is not exhaustive as it does not include some factors such as the costs of drying and further processing of the biofuel, the cost of packaging and marketing nor the cost of capital of this stage because of the lack of  data about it, so the focus will be on calculating the operation cost for the final product, depending on assumption and input data listed in Table 6 , to develop the equation as listed below :

| Details | Equation | No |
| --- | --- | --- |
| Total capital cost | TCAP.C=TPA×(C.SIP+C.CLS+ENG.F+COG+C.LAND) | 1 |
| Arial productivity at various levels of (Ph). | AR.P=VP× (PD×1000) | 2 |
| Annual productivity at various levels of (Ph) | AN.P=VP×TGD×(TPV×1000)×(1-PDW)×PPH×HE/ 1000000 | 3 |
| The annual extra volume of culture media to replete the amount of lost | AEV=TPV×PPH×TGD× (1-HE) | 4 |
| The nutrients added to make up for it take up by cells | NARC=NUC×PPH×TGD(TPV×1000) | 5 |
| The amount of nutrients added to the medium | NAHC=NUC×TGD×PMR×(TPV×1000)×(1-PPH) | 6 |
| The amount of nutrients required for making up the extra culture media | NPE=AEV×NCM | 7 |
| The amount of nutrients added per year. | NGN=NARC+NAHC+NPE/1000 | 8 |
| Total annual cost for co2 | TCdifferent ph =COC×CNdifferent ph×(TPV×1000)×(1-PDW) | 9 |
| Annual cost for total fresh water added | TFWC=TPA×ARE×AR | 10 |
| Labour cost | TLC=LCS+LCST+LCTD+LCTS | 11 |
| Annual power (Electricity) cost | APOC=24×POC×POU×TGD×TPHA×(1-PDW) | 12 |
| Cost before harvesting | PUPH=(TCAC/12)+ANC+APC+TFWC+Asco+TLC+ TPOC/AVPph ×1000 | 13 |
| Cost after harvesting | PHPH=(TCAC/12)+ANC+APC+TFWC+Asco+TLC +TPOC + AHC/AVPph×1000 AHC=From Table :Harvesting days/year=140 day :VH=TPV×PPH×TGD×(1-PDW) | 14 |
| Rate of production lipids | BP=(((fcl*Pa)/ρcl)*length*width*No of ponds)/3.75/45 Pa=Arial productivity (kg/m2.y) : fcl=dry mass microalgae lipid fraction: ρcl=lipid density(kg/l) | 15 |
| Indirect costs | (Insurance=TCAC×2%)(Depreciation=TCAC×10%) (Maintenance = TCAC× 3%)(Others = TCAC ×1%) | 16 |
| Oil yield (gallon & barrel) | Operation($/ton)=operation/AVP : Noha=PL*w*pnum/10000: tpap = PL * PW * PNUM: oilpp = (tpap * oilp)/37.85/3.9: oilppb=oilpp/45 | 17 |
| Revenue | R= (oilppb * price)/AVP Oilppb= oil yield : AVP=annual productivity | 18 |
| benefit | B=Revenue-operationX OperationX=operation cost ($/ton) | 19 |

Table 6. Assumption and Input data of biofuel cost process

|  | Case | Abbreviation | Value | Units |
| --- | --- | --- | --- | --- |
| 1 | Cost of Site preparation | C.SIP | 16000 | $/ha |
| 2 | Cost of Culture system | C.CLS | 240000 | $/ha |
| 3 | Engineering fee | EF | 38400 | $/ha |
| 4 | Contingency | COG | 12800 | $/ha |
| 5 | Cost of Land | C.LAND | 1000 | $/ha |
| 6 | Land / Area(ha) | LAND/Areal | 100, 200, 300, 400, 500 |  |
| 7 | Total growth days | TGD | 330 | day |
| 8 | Proportion of down time | PDW | 15% | % |
| 9 | Proportion of pond harvesting | PPH | 50% | % |
| 10 | Cost of nitride | ANC | 1 | $/kg |
| 11 | Cost of phosphuors | APC | 1.5 | $/kg |
| 12 | Harvesting efficiency | HE | 90% | % |
| 13 | Pond depth | PD | 0.2 | M |
| 14 | Pond length | PL | 530 | M |
| 15 | Pond width | PW | 12 | M |
| 16 | No of ponds | No.P | 157,314,471,628,785 | - |
| 17 | Used nitride from culture media | NUC | 0.0075 | g/l |
| 18 | Proportion of medium recycled | PMR | 90% | % |
| 19 | Algae doubling time | td | <2 | day |
| 20 | NaNo3 concentration in medium | NCM | 0.075 | g/l |
| 21 | Used phosphors from culture media | PUC | 0.0003 | g/l |
| 22 | NaH2PO4.1 H2O concentration | PCM | 0.005 | g/l |
| 23 | Volumetric productivity | VP | 0.078 | g/l.d |
| 24 | Co2 cost | COC | 0.068 | $/l |
| 25 | Co2 required for different ph | CNph | 0.35 | l/d |
| 26 | Fresh water cost | FWC | 0.05 | $/m3 |
| 27 | Average days of evaporation | ADE | 330 | D |
| 28 | Rate of evaporation | RE | 0.03 | M |
| 29 | Average rainy days | ARD | 25 | D |
| 30 | Average rain | AR | 0.1 | m.d-1 |
| 31 | Labour cost supervisor | LCS | 2058.33 | $/ha.y |
| 32 | Labour cost senior technision | LCST | 2816.66 | $/ha.y |
| 33 | Labour cost technision-day term | LCTD | 6933.33 | $/ha.y |
| 34 | Labour cost technision-shift term | LCTS | 3900 | $/ha.y |
| 35 | Power cost | POC | 0.15 | $/ha.y |
| 36 | Power usage | POU | 65 | $/kw.hr |
| 37 | Harvesting system cost annually | AHC | From Table.2 (.35) | $ |
| 38 | Oil lipid | oilp | (5-60) Taken (30) | $ |
| 39 | Price /barrel of Oil | price | 150 | $ |

Note: Barrel=42 gallon (according to us measurement) was the adoption of info@digitaldutch .com

But we noted that any factor can increase the net productivity of the plant on annual biases at little incremental capital or operating cost, is of fundamental importance. The simplest and most important of these are:

- Site selection. A microalgae plant should be located at a site where climatic conditions provide optimal growth conditions for the longest possible period.

- Selection for faster growing, more productive strains optimized for the prevailing climatic conditions is also important. To do this it essential to have a good understanding of those factors which limit growth and productivity, an excellent example of such studies is given in the paper by Vonshak and coworkers (Richmond et al., 1980; Vonshak et al., 1982, 1983; Vonshak, 1987).

- Culture systems :productivity can also be improved through better culture systems, however the incremental cost of these culture systems in relation to the improvement in productivity must be evaluated carefully.

- The cell concentration of the products is also very important. Increased product concentration not only decreases the effective unit cost of the raw biomass, but it also generally reduces the cost of extraction and purification.

- Harvesting system which represents a significant capital and operation cost component. So the choice of harvesting system also depends on final product desired. It is therefore desirable to select an algae with properties which simplify harvesting i.e. large cell size, high specific gravity in comparison to the medium. The cost of harvesting may be offsite against the cost of achieving higher productivity.

- Algal-biofuel production labour: Labour is required for pond and equipment maintenance, monitoring of the cultures, harvesting, extraction and further processing. So any improvements in the design of the process and automating the operations of the plant which decrease the labour requirement, without unreasonably increasing the capital costs need to be considered carefully as a possible means of reducing production costs.

For commercial production the whole process of culture, harvesting and subsequent downstream processing must be reliable. particularly in case of  large open pond systems , open air cultures in use today. These assume a good understanding of the algae and their interaction with the environment. For this the direction towered Closed culture systems, such as the tubular photo bioreactors which are being developed in several parts of the world (Chaumont et al., 1988; Borowitzka & Borowitzka,1988, 1989; Hoshino et al., 1991) are another way to improve reliability since the growth environment can be much more closely monitored and controlled.

4-  Discuss the results

The main goal of this research is to measure the cost of barrel of algae-biofuel in comparison with the cost of barrel of traditional oil, so in this paper we gain the result as shown in Table 8 after applying a theoretical approach to evaluate economics & cost–benefit of product biofuel from microalgae.

The results show that the cost of barrel of algae-biofuel is still higher than the cost of barrel of traditional oil, but this result was much better than the result of others like Harmelen (336,525$resp) (Harmelen .T.V, 2006),  the cost of barrel between (143.88-151.51$), it can be reduce for commercial purpose if improved the harvesting efficiency and increase the productivity of algae oil content of species. The total cost of algae process is sensitive to culture area and it so much sensitive to the productivity and improving the harvesting efficiency. The operation cost per unit (if productivity $/ton) of biomass is increased with the increasing area until reaches 300 hectare, then it begun to decrease ,this means that the commercial cost be benefit with the increasing the culture area. The benefit and revenue increased until 400 ha and  jumped sharply until 500 ha of cultural area, this means that the good benefit confirmed at 500 ha of area. The analysis shows that the operation cost components like water cost are of high major significance. The cost of the biomass is most sensitive due to increases in labour and power costs, whereas nutrient has little impact, which much better from the result of (Phillip Brown) as shown in Figure 4. Once potential contributor to overall operating costs, is the cost of addition of CO2 to the culture. In open systems at normal pH & pH<9 the transfer efficiency of CO2 is low, and it is doubtful whether the increased productivity, algal production is intensive to Labour which is required for pond and equipment maintenance, monitoring of the cultures, harvesting, extraction and further processing .Any improvements in the design of the process and automating the operations of the plant which decrease the labour requirement.

Table 7. The Output

| Area(ha) Parameters | 100 | 200 | 300 | 400 | 500 |
| --- | --- | --- | --- | --- | --- |
| Capital cost ($) | 3.077E+07 | 6.154877E+07 | 9.232316E+07 | 1.230975E+08 | 1.538719E+8 |
| Annual productivity (Ton/y) | 1966.196 | 3932.392 | 5898.587 | 7864.783 | 9830.979 |
| Total cost ($) | 3.02E+07 | 6.1E+07 | 9.08E+07 | 1.2E+08 | 1.5E+08 |
| Operation cost ($) | 2.785418E+5 | 5.570837E+05 | 8.356255E+05 | 1.114167E+06 | 1.392709E+6 |
| Biomass cost ($/ton) = operation cost /Annual productivity | 137 | 139 | 140 | 139 | 132 |
| Biomass cost ($/kg) = Operation cost / Annual productivity | 0.137 | 0.139 | 0.14 | 0.139 | 0.13 |
| Cost before harvesting ($/kg) | 4.06 | 2.031767 | 1.354511 | 1.015883 | 0.8127068 |
| Cost after harvesting ($/kg) | 6.59 | 3.297375 | 2.19825 | 1.648687 | 1.31895 |
| Biodiesel productivity (gallon/y) oil yield = 30 | 202930 | 405861 | 608791 | 811722 | 1014563 |
| Biodiesel productivity (barrel/y) oil yield=30 | 4509 | 9019 | 13528.7 | 18038 | 22547 |
| Biodiesel productivity(barrel/ton) | 2.3 | 2.2 | 2.3 | 2.3 | 2.3 |
| Barrel cost ($/barrel) add 9% of biomass cost as extraction process cost | 150 | 151.51 | 152.6 | 151.51 | 143.88 |
| Gross annual revenue ($/ton) | 344 | 344 | 344 | 344 | 344 |
| Profit ($/ton) (Revenue- Biomass cost) | 207 | 205 | 204 | 205 | 312 |
| Profit (Revenue- Barrel cost) | 194 | 192.49 | 191.6 | 192.49 | 200.12 |
| Cost of gallon ($) | 3.57 | 3.6 | 3.62 | 3.6 | 3.42 |

Table 8. Operation cost driver for various items of cultural areas

| Details | 100,200,300,400,500(Hectares) |
| --- | --- |
| Contribution of nitride to total operation cost | 1.68 % |
| Contribution of phosphors to total operation cost | 0.1% |
| Contribution of CO2 to total operation cost | 14.5% |
| Contribution of labour to total operation cost | 5.6% |
| Contribution of power to total operation cost | 23% |
| Contribution of water to total operation cost | 17% |

Figure 3. Operation items cost drivers  1- Nitride, 2- Phosphorus, 3- CO2, 4- Labour, 5- Power, 6- Fresh water.

*(Brown. P, 2009)

Figure 4. Algal biofuels economic drivers.

5- Conclusions and Comparison with the other workers

The applying cost structure model indicated that there was economic feasibility from produced algal-bioful ,although the production cost was high but it was reasonable cost in comparing it with the damage and costs spent to protect the environment from pollution, others conclusion was the cost and revenue sensitive to the productivity and culture area which agree with results of other researchers as shows in  Table 9,Figure 5.

Table 9. Comparison between several workers for many algae variable parameters

| Algae Type | Culture System | Culture area/volume | Productivity | Cost $/Kg | Reference |
| --- | --- | --- | --- | --- | --- |
| Scenedesmus | Raceway | 4 ha | 20 | 7 | Becker et al. (1980) |
| Spirulina | Ponds | 20 ha | 12 | 5 | Rebeller(1982) |
| Spirulina | Raceway | 5 ha | 12 | 12 | Rebeller(1982) |
| Microalgae | Ponds | 800 ha | 17 | 0.4 | Benemann et al.(1982) |
| Spirulina | Raceway | 10 ha | 14 | 6 | Richmond (1983) |
| Microalgae | Ponds | 10000 ha | 20 | 0.3 | Regan&Gartside (1983) |
| Porphyridium | Solar water | 34 ha | 30 | 14 | Anderson&Eakin(1984) |
| Cruentum | Heater | - | - | - | Tapie&Bernard (1958) |
| Microalgae | Tubular reactor | 10 ha | 16 | 7 | Tapie&Bernard (1958) |
| Chlorella | Raceway | - | 25-30 | 22 | Kawaguchi (1980) |
| Chlorella | Raceway | - | 25-30 | 23 | Kawaguchi (1980 |
| Spirulina | Raceway | 5 ha | 3.2 | 22 | Jassby (1988) |
| Dunaliella | Raceway | 2 ha | 4 | 10 | Mohn&Cordero (1990) |
| Microalgae | Tank culture | 20 m3 |  | 60 | Fulks&Main (1991) |

Figure 5. Shows the effect of area size from 1-32 ha on algae productivity and the final production cost ($/kg), the productivity increased while cost rate decreased to lower levels.

Refe     rences:

- International Energy Agency World Energy Outlook (2007). China and India Insights. International Energy Agency Publications; Paris, France.

- International Energy Agency (2008).From 1st -2nd –Generation Biofuel Technologies. An overview of current industry and RD&D activities, OECD/IEA, November, 2008.

- http://www.altprofits.com/ref/report/biofuels/ biofuels.html

- Antoni, D.; Zverlov, V. V.; Schwarz, W. (2007) H. Biofuels from microbes Appl. Microbial. Biotechnology. 77 (1) 23– 35

- Srivastava A, Prasad R. (2000) Triglycerides-based Diesel Fuels. Renew Sust Energy Rev. 4:111–133.

- http:WWW.oilgae.com/algae/algae.htm, introduction to algae types of algae.

- (WWW.algaebase.org).

- Report of GBEP. (2009), Algal-BASED BIOFUELS.A Review of Challenges and Opportunities for Developing Countries, May.

- Report of NREL, (1998). A Look Back at the U.S. Department of Energy’s Aquatic Species Program: Biodiesel from Algae. National Renewable Energy Laboratory, U.S. Department of Energy’s, Office of Fuels Development.

- Everett J. Nichols, Ph.D., MSPH: Scientific Director of Biopolymer Research (Water Treatment Methods for Control and Management of Algae.

- Christi, Y. (2007).Biodiesel from microalgae, Biotechnology Advance, 25:294-306.

- Jackson DD, Ellms JW (1896) On odors and tastes of surface waters with special reference to Anabaena, a microscopial organism found in certain water supplies of Massachusetts, Rep. Mass.State Board Health ,410–420.

- Oswald, W.  J. and C .G. Golueke. (1960) Biological Transformations of Solar Energy, Advances in Applied Microbiology, 25:692-704.

- Burlew, J. S. (1953) Algae culture from laboratory to pilot plant. Carnegie Inst. Wash. Publ. No. 600.

- Fisher, R.A. (1935). The Design of Experiments, Oliver and Boyd, Edinburgh and London.

- Fisher, A.W, (1955)"Engineering for Algae Culture". Solar Energy 0 Research (Daniels and Duffie, eds), University of Wisconsin Press, Madison, WI.

- Oswald, W.J., and Golueke, C.G. (1960). Adv. Appl. Microbiol., 11, 223 – 242.

- Benemann, J., B. Koopman, J.C. Weissman, D.M. Eiseberg and W.J. Oswald, 1977. Cultivation on sewage of microalgae harvestable by microstrainers ( contract Nos, W-74-05 EnG-48 and E-(04-3)-34). Prepared for US Energy Research and Development Administration.

- Soeder, C. J., M. E. Meffert, I. Rolle, W. Pabst, H. D. Payer, and E. Stengel.(1975) "Das Dortmunder Verfahren Zur Produktio-nEssbarer Mikroalgen, Kohlenstoff Biologische Forschungstation E.V., Dortmund, Germany, 1975.

- Catie, R. Terrapin, B,G,.LLT ( 2009) Cultivating Clean Energy,The Proise Of Algae Biofuels , Natural Resources Defense Council.

- J. Benemann, (2008) 5th Annual World Congress on Industrial Biotechnology, Chicago, April 30.

- RichmondA, Vonshak A, Arad S (1980) Environmental limitations in outdoor production of algal biomass. Elsevier/ North Holland Biomedical Press, Amsterdam, 62-72.

- Vonshak  A , Abeliovich A, Boussiba S, Arad S, Richmond A(1982) Production of Spirulina biomass: effects of environmental factors and population density. Biomass 2: 175-185.

- Vonshak A, Boussiba S, Abeliovich A, Richmond A (1983) Production of Spirulina biomass: maintenance of monoalgal culture outdoors, Biotechnol. Bioeng.25: 341-349.

- Vonshak A (1987) Strain selection of Spirulina suitable for mass production, Hydrobiologia 151/152: 167-172.

- Chaumont D ,Thepenier C, Gudin C,J unjas C (1988) Scaling up a tubular photoreactor for continuous culture of Porphyridium cruentum from laboratory to pilot plant (1981-1987). In Stadler T, Mollion J, Verdus MC, Karamanos Y, Morvan H, Christiaen D (eds), Algae Biotechnolology. Elsevier Applied Science, London, 199-208.

- Borowitzka, M.A.(1988a) Appendix: Algal media and sources of algal cultures.In:Micro-algal Biotechnology.( Borowitzka,M.A. and . Borowitzka, L.J.eds). Cambridge University Press, Cambridge, PP: 456-465.

- Borowitzka, M.A.(1988b) Fats, oil and hydrocarbons, In:Micro-algal Biotechnology.( Borowitzka, M.A. and Borowitzka, L.J. eds). Cambridge University Press, Cambridge, PP: 257-287.

- Borowitzka L J, Borowitzka M A (1989) Industrial production:methods and economics. In Cresswell RC, Rces TAV, Shah N (eds), Algae and Cyanobacterial Biotechnology, Longman Scientific, London, 294-316.

- Hoshino K, Hamochi M, Mitsuhashi S, Tanishita K (1991) Measurements of oxygen production rate in flowing Spirulina suspension. Appl. Microbiol. Biotech. 35: 89-93.

- Harmelen TV,Oonk H (2006) Microalgae biofixation processes: Applications and potential contributions to greenhouse gas mitigation options ,prepared for the international network on biofixation of CO2 and green house gas abatement with microalgae operated under the international energy agency greenhouse gas R&D programme, www.tno.nl    .

- Brown.P (2009) Algal Biofuels Research, Development, and Commercialization Priorities: A Commercial Economics Perspective, www.diversified-energy.com .

- Backer, E.W. and L.V. Venkatamaran, 1980. Production and Processing of Algae in Pilot Plant Scale: Experiences of the Indo-German Project. In: Algae Biomass: Production and Use, Schelef, G. and C.J. Soeder (Eds.), Elsevier/North Holland Biomedical press, Amsterdam, pp: 35-50.

- Rebeller M., (1982) Techniques de culture et de recolte des algues spirulines. Doc. IFP: 1-14.

- Benmann JR,Goebel RP,Weissman JC, Augenstein DC (1982) microalgae as a source of liquid fuels. Report to DOE office of energy Research May 1982:1-17

- Richmond, A., 1983. Phototrophic Microalgae. In: Biotechnolology, Rehm, H.J., G. Reed and H. Dallweg (Eds.), Verlag Chemie, Weinheim, pp:109-144.

- Regan, DL.,Gartside,G.,(1983) Liquid fuels from micro-algae in Australia. CSIRO, Melbourne,pp55.

- Tapie, R and Bemard, A. (1987) Microalgae production: technical and economical evaluations. Biotechnology and Bioengineering. 32:873-885.

- Kawaguchi, K., 1980. Microalgae Production Systems in Asia. In: Algae Biomass Production and Use, Shelef, G. and C.J. Soeder (Eds.). Elsevier/North Holland Biomedical Press, Amsterdam, pp: 25-33.

- Jassby, A., 1988. Spirulina: A Model for Microalgae as Human Food. In: Algae and human Affairs, Lembi, C.A. and J.R. Waaland (Eds.). Cambridge University Press, Cambridge, New York, pp: 149-179.

- Mohn, F. H. and Cordero, O. (1990) Harvesting of the algae Dunaliella some consideration concerning the cultivation and impact on the production cost of B-carotene. Berichte des Forschungszentrum Julich. 2438: 1-50.

- Fulks W,Main Kl (1991) Rotifer and Microalgae culture systems.The Oceanic In statute,Honolulu,364pp.

Programming of modeling process

50 INPUT "SPREP =Site Preparation($/ha)** ="; SPREP INPUT "CSYS =Culture System ($/ha)******* ="; CSYS INPUT "ENGF =Engineering Fee ($/ha)****** ="; ENGF INPUT "CONTING =Contingency ($/ha)** ="; CONTING INPUT "Land =Land ($/ha) ************* ="; LAND INPUT "TGD=Total growth days*********** ="; TGD INPUT "PDW= Proportion of down Time*** ="; PDW INPUT "PPH= Proportion of Pond Harvesting* ="; PPH INPUT "Ncost=cost of nitride/kg ********* ="; Ncost INPUT "Pcost=cost of phosphorate/kg ***** ="; Pcost INPUT "HE=Harvesting Efficiency (%)******** ="; HE INPUT "ODEPTH=Pond Operating Depth (m)* ="; ODEPTH INPUT "LENGTH=Pond Operating LENGTH (m)* ="; LENGTH INPUT "WIDTH=Pond Operating width (m)**** ="; W INPUT "PNUM= No of ponds ************* ="; PNUM INPUT " NUC=Used Nutriedes from culture media* ="; NUC INPUT "PMR=Proportion of medium recycled**** ="; PMR INPUT "td =Algae Doubling Time < 2 days******** * ="; td INPUT "NCM=NaNo3 concentration in medium*** ="; NCM INPUT " PUC=Used phosphorus rate from culture media* ="; PUC INPUT " PCM=NaH2PO4 .1H20 Concentration******** ="; PCM INPUP"VP=Volumetric productivity of different (ph)(g/L.day)* =";VP INPUT "CC=CO2 Cost ******** *** ="; CC INPUT "CN ph=CO2 required fore different PH** ="; CNph INPUT "FWC = Fresh water cost ($/m3) ******** ="; FWC INPUT "ADE=Average days of evaporation (day** ="; ADE INPUT "EV=Rate of evaporation (m) ************ ="; EV INPUT "LCS =Labour cost supervisor *********** ="; LCS INPUT "LCST=Labour cost senior technician ***** ="; LCST INPUT "LCTD=labour cost technician-day term ** ="; LCTD INPUT "LCTS=labour cost technician-shift term * ="; LCTS INPUT "POC=Power Cost ******************** ="; POC INPUT "POU=Power Usage ****************** ="; POU INPUT "AHC=Harvesting system cost ($)/Table*** ="; AHC1 INPUT"fcl=Dry mass microalgae lipid content fraction* =";fcl INPUT"dcl=Density of lipids usable for conversion to biodiesel(kg/L* =";dCL INPUT "price ($) ********************** ="; price REM ********* Total Capital Cost ************** TPHA = LENGTH * W * PNUM / 10000 TPV = ODEPTH * LENGTH * W * PNUM TPA = LENGTH * W * PNUM TCAP = TPHA * (SPREP + CSYS + ENGF + CONTING + LAND) REM *********** Arial Productivity **************** AP = VP * (ODEPTH * 1000) REM **** Biodiesel productivity/year...(gallon& barrel) ***** PA=(AP*360)/1000 PCL1=(fcl*PA)/dcl PCL=(PCL1*TPA)/3.75*10 PCLL=PCL/45 REM ****** Annual Productivity ************** x = 1 - PDW AVP = (VP * TGD * TPV * 1000 * x * PPH * HE) / 1000000 AVP1 = AVP * 1000 REM ************ AN cost ********************* AEV = TPV * PPH * TGD * (1 - HE) NARC = NUC * PPH * TGD * (TPV * 1000) NAHC = NUC * TGD * PMR * (TPV * 1000) * (1 - PPH) NPE = AEV * NCM NGN = (NARC + NAHC + NPE) / 1000 ANcost = NGN * Ncost REM ******* APcost ********************** PARC = PUC * PPH * TGD * (TPV * 1000) PAHC = PUC * TGD * PMR * (TPV * 1000) * (1 - PPH) PPE = AEV * PCM PGp = (PARC + PAHC + PPE) / 1000 APcost = PGp * Pcost REM ***************************** CO2 Cost ********************************* TC = CC * CNph * (TPV * 1000) * (1 - PDW) REM ***************************** Fresh water Cost *************************** X11 = ADE * EV TPV1 = X11 * TPA TPV2 = TPV - TPV1 TFWA1 = TPV2 * FWC TFWA = -TFWA1 REM ***************************** Labour Cost ******************************* Noha=length*w*pnum/10000 LCS1=LCS*Noha LCST1=LCST*Noha LCTD1=LCTD*Noha LCTS1=LCTS*Noha TLC = LCS1 + LCST1 + LCTD1 + LCTS1 REM ***** Power Cost ************ TPOC = 24 * POC * POU * TGD * TPHA * (1 - PDW) REM ******* Cost before Harvesting *********** PUPH = ((TCAP / 12) + ANcost + APcost + TC + TFWA + TLC + TPOC) / (AVP * 1000) PUPH1 = TCAP + ANcost + APcost + TC + TFWA + TLC + TPOC cbefore = (ANcost + APcost + TC + TFWA + TLC + TPOC) / (AVP * 1000) cbefore1 = ANcost + APcost + TC + TFWA + TLC + TPOC REM ***************************** Cost After Harvesting ************************ AHC=AHC1*TPV*140 PHPH = ((TCAP / 12) + ANcost + APcost + TC + TFWA + TLC + TPOC + AHC) / (AVP * 1000) PHPH1 = TCAP + ANcost + APcost + TC + TFWA + TLC + TPOC + AHC cafter = (ANcost + APcost + TC + TFWA + TLC + TPOC + AHC) / (AVP * 1000) cafter1 = ANcost + APcost + TC + TFWA + TLC + TPOC + AHC REM ***************************** Total Cost before+ After Harvesting ************ Insurance = TCAP * .02 depreiation = TCAP * .1 maintanence = TCAP * .03 Others = TCAP * .01 indirectcosts = insurance + depreiation + maintanence + others x = (PUPH / AVP)*1000 Y = (PHPH / AVP)*1000 Z = ((PUPH + PHPH) / AVP)*1000 TOTAL=PHPH1+indirectcosts OPERATION=cafter1+indirectcosts j1= (ANcost/operation)*100 j2= (APcost/operation)*100 j3= (TC/operation)*100 j4= (TLC/operation)*100 j5= (TPOC/operation)*100 j6= (TFWA/operation)*100 REM ..................OIL YIELD / {GALLON/BARREL} ************************************ Noha=length*w*pnum/10000 REM *************tpap = LENGTH * W * PNUM REM ****** oilpp = (tpap * oilp)/37.85/Noha*2.5 REM ********************** oilppb=oilpp/45 Revenue= PCL * price Benefit=Revenue-operation REM $$$$$$$$$$$$$$$$$ OUTPUT $$$$$$$$$$$$$$$$$$$$ PRINT "TCAP=TOTAL CAPITAL COST ($) ******* ="; TCAP PRINT "AVP=ANNUAL PRODUCTIVITY *(Ton/year)******* ="; AVP PRINT "ANcost=Annual (N) cost ($/y) *********** ="; ANcost PRINT "APcost=Annual (P) cost ($/y) ***** ="; APcost PRINT "TC=Total Annual cost for co2 ($/y) ********* ="; TC PRINT "TFWA=Total Fresh water added by rain ($/y) **** ="; TFWA PRINT "TLC= Total labour cost ($/y) ********** ="; TLC PRINT "TPOC=Total power cost for ponds operation ($/y)* ="; TPOC print"TOTAL ($) ********* =";TOTAL PRINT"operation ($) ****** =";operation PRINT "X ($/unit) *********** ="; x PRINT "Y ($/unit) ************ ="; Y print "Noha= number of hectars ********** =";Noha print"PCL=biodiesel productivity(gallon/year) ************ =";PCL Print "PCLL=biodiesel productivity (barrel/year) *********** =";PCLL PRINT "Gross annual revenue ($/y) ************** ="; Revenue print"benefit ($/y) *************** =";benefit print"j1= contribution of nutride cost to total operation cost(%)* =";j1 print"j2= contribution of phosphours cost to total operation cost(%)* =";j2 print"j3= contribution of co2 cost to total operation cost(%) ** =";j3 print"j4= contribution of labour cost to total operation cost(%)* =";j4 print"j5= contribution of power cost to total operation cost(%)* =";j5 print"j6= contribution of water cost to total operation cost(%) * =";j6 GOTO 50