Computer Architecture

33d  stage

30-11-2011

College of sciences for women ((())) Dept. of computer sciences

Year 2011-2012

Lec. (7)

6.5 Control-Flow

So far we have only considered programs, i.e., sequences of instructions, whose control-flow is a straight line. That is, the flow of execution starts at the top and proceeds through each instruction in sequence before ending at the bottom. In reality, this places a massive restriction on the types of programs we can write. For example we cannot conditionally execute sequences of instructions based on some input or intermediate result, or iterate over a sequence of instructions to form what we commonly call loops. Providing the ability to alter the control-flow of a program is therefore a vital part of both instruction set and micro-architecture design. This is particularly true since one can view instructions that alter control-flow as pure overhead: they are not really performing any useful computation themselves, just selecting which computation is performed. As such, ensuring that the control-flow mechanism does not have a significant impact on performance is also vital.

Roughly speaking, an instruction which might alter the flow of execution is called a branch instruction. Any branch will achieve a change in control-flow by updating the value of PC, the program counter, which holds the address of the next instruction to be executed. We can classify branches according to a number of criteria which dictate how and when they perform this update:

Conditional Versus Unconditional A branch is unconditional if it always updates the program counter; such a branch is sometimes called a jump. A branch is conditional if the update only occurs when some condition is true. Typically the condition will be a comparison between operands. When a branch is executed so that we know where next to fetch instructions from, i.e., the processor knows if it is taken or not, we say the branch has been resolved.

Examples of both forms are easy to see in the MIPS32 ISA. For example, the j instruction is an unconditional branch, it simply sets the PC to equal the immediate operand. Both beq and bltz are conditional branches since they compare two values to each other, two registers or a register and an immediate respectively, and only update the PC if the result of the comparison is true.

Absolute Versus Relative An absolute branch sets the PC to a value unrelated to the value it currently holds; the instruction simply replaces the value in PC with some new address. A PC relative (or offset) branch updates the PC by changing it by some amount (i.e., adding or subtracting some value); the end result is that the branch is relative to the PC rather than just any address.

Most branches are fairly local, it is uncommon for a branch to update the PC with an absolute address. As such, one can encode the target address more compactly as an offset from the PC than a full 32-bit address. This means that, for example, a single MIPS32 conditional offset branch beq can include a 16-bit immediate and cope with most offsets required.

There is one caveat to offset branches, which is related to when the branch is actually executed. In the fetch-decode-execute cycle we already said that the PC would be incremented by some amount during the fetch stage so it points to the next instruction to execute. By the time the processor enters the execution phase for a taken branch, we are essentially saying that this is not actually the instruction we want to execute at all, some other one is. But the PC has already been incremented so we need to take this into account when calculating the offset: typically the offset required will be one instruction, four bytes in the MIPS32 case, less than one might have thought. If this off-by-one problem is not taken into account, the processor will branch to the wrong place !. Figure 5 attempts to demonstrate the difference between absolute and offset branches; both Figure 5a and Figure 5b have two identical branches at addresses n and m, the result of their execution is different however. In particular, both branches in Figure 5a result in control-flow being sent to address 100 while in Figure 5b the branch target depends on what address the branch instruction resides at.

Figure 5 The difference between absolute and offset (or PC relative) branches.

Immediate Versus Computed The memory in a typical 32-bit processor has 232 addressable locations. To branch to an arbitrary address therefore, one has to provide a 32-bit address. Encoding a 32-bit address inside a 32-bit instruction is impossible: there is no space for the opcode so some compromise must be made. The unconditional absolute branch j is a good example of this, it sets the PC to equal a 26-bit immediate which cannot address the whole memory but does cover a fairly large region.

To cope with the need to branch to arbitrary addresses, most ISAs include a computed branch instruction which takes the new PC value from a register operand. Since the register can contain a full 32-bit value, the branch can be to anywhere in memory. The trade-off is that one has to execute more instructions in order to compute this address in the register before the branch is executed; with immediate branches these extra instructions are not required.

Other than these different patterns of actual behavior, the rest of the fetch-decode-execute cycle is unchanged. The completion of a branch in the execution stage means the right value is in the PC for the next fetch stage; the cycle simply carries on as normal except that we have altered where in the program we are now fetching instructions from.

One slight subtlety introduced by the MIPS32 ISA is the idea that we need not store the whole branch target as an immediate in the instruction encoding. Each instruction is a 32-bit value so if the PC starts at address zero, instructions always start on a multiple of four bytes; they are word aligned. If we are using a byte addressable memory where each byte has an address, the addresses of instructions are all 0 modulo 4, which is to say the least-significant two bits of the address are always zero. Since the PC is always word aligned, any offset from it will also be a multiple of four bytes and hence will also always have the least-significant two bits as zero. Given this fact, we do not need to store the bits in an encoded instruction.

For example, the meaning of the j instruction is

PC←(imm : 00(2))

where imm is a 26-bit immediate stored in the instruction. This 26-bit immediate has the two implicit zero bits appended to the least-significant end via the concatenation operator before it is used to update the PC. This way, our 26-bit immediate does not waste space, and can specify a larger range of addresses as a result.

Table 1 A tabular description of a reduced MIPS32 ISA.