Filler Types

The use of fillers in rubber products is nearly as old as the use of rubber itself. In time, the fillers which were likely first used for their color, zinc oxide and carbon black, became recognized for their reinforcement potential. In the early years of the 20th century, prior to World War I, zinc oxide was the most widely used reinforcing filler in rubber; the abrasion resistance it provided made it the preferred filler in tire treads. It was also during this

period that zinc oxide was discovered to be the activator for the newly emerging organic accelerators. The reinforcing effect of carbon black was quantified during the first decade of this century, but remained unexploited for about ten years.

Carbon Black – Carbon black is essentially elemental carbon in the form of fine amorphous particles. Each particle is composed of randomly oriented microcrystalline layered arrays of condensed carbon rings. Because of their random orientation, many arrays expose open layer edges with unsatisfied carbon bonds at the particle surface. This in turn provides the sites for chemical activity. Individual round carbon black particles do not exist as discrete entities but form aggregates, which may be clumps or chains of various sizes and configurations. The functional carbon black “particle”, therefore, is actually the aggregate. Average particle size and aggregate configuration (structure) are the major determinants of the utility of a given carbon black in a specific rubber compound. The major differences among commercial grades result from control of these averages.

Prior to World War II, the predominant reinforcing black was made from small natural gas flames impinging on iron channels. The deposit scraped from these channels was known as channel black. The finest particle size used for rubber, about 24 nm average, was Hard Processing Channel (HPC) because it produced the stiffest stocks. Larger particles sizes were available as Medium Processing Channel (MPC) at 26 nm average, and Easy Processing Channel (EPC) at 29 nm average. Channel blacks are no longer used in rubber because of their high cost and the availability of suitable alternatives.

Furnace blacks began displacing channel blacks for rubber reinforcing in the early 1940s. Furnace blacks were at first, as early as the 1920s, made by burning natural gas in large horizontal furnaces, yielding a relatively coarse (60 to 80 nm average) semi-reinforcing product.

The thermal process, introduced in 1922, makes the largest particle size and lowest structure blacks. Thermal blacks are made in a large cylindrical furnace by the thermal decomposition of natural gas in the absence of flame or air. Thermal blacks range from 100 to 500 nm average and are generally used as low cost functional extender fillers. Their relatively large size and low structure enable higher loadings and provide better resilience and lower hysteresis than the more reinforcing blacks.

The carbon blacks that provide the highest ratio of reinforcement to surface area are those produced using the shortest reaction time. Short reaction time promotes randomness of carbon ring layer orientation within the particle and the consequent occurrence of layer edges, with unsatisfied carbon bonds, at the surface.

Until 1968, carbon black nomenclature was informal and based on a variety of characteristics, including level of abrasion resistance, level of reinforcement, vulcanizate modulus, processing properties, general usefulness, particles size, and electrical conductivity. In 1968, the ASTM Committee on Carbon Black established a common nomenclature system consisting of a prefix followed by a three digit number. The prefix is either N, for normal curing, or S for slow curing.

The first of the three digits indicates a range of average particle size in nanometers. The second and third digits are assigned by the ASTM Committee to new products as they are developed. In general, lower structure blacks are assigned lower numbers and higher structure blacks, higher numbers, although there are some exceptions. Table 2 shows carbon blacks by ASTM classification, the old letter classification and size ranges.

Kaolin Clay – Kaolin clay is typically used to reduce rubber compound cost while improving physical or processing properties. Rubber filler clays are classified as either “hard” or “soft” in relation to their particle size and stiffening affect in rubber. A hard clay will have a median particle size of approximately 250 to 500 nm, and will impart high modulus, high tensile strength, stiffness, and good abrasion resistance to rubber compounds. Soft clay has a median particle size of approximately 1000 to 2000 nm and is used where high loadings (for economy) and faster extrusion rates are more important than strength.

The anisometry (planar shape) and particle size of the clays account for their affect on modulus and hardness. More hard clay than soft is used in rubber because of its semi-reinforcing effect and its utility as a low cost complement to other fillers. It is used to improve the tensile and modulus of ground calcium carbonate compounds and will substitute for a portion of the more expensive carbon black or precipitated silica in certain compounds without sacrificing physical properties.

Some water-washed clay is used because its lower level of impurities provides better color and less die wear with extrusions. Aminosilane and mercaptosilane treated hard clays provide better reinforcement than untreated clay, and in some applications can rival furnace blacks.

Calcium Carbonate – Calcium carbonates for rubber, often referred to as “whiting”, fall into two general classifications. The first is wet or dry ground natural limestone, spanning average particle sizes of 5000 nm down to about 700 nm. The second is precipitated calcium carbonate (PCC) with fine and ultrafine products extending the average size range down to 40 nm.

Ground natural products show low anisometry (specific shape depends on grinding process), low surface area and low surface activity. They are widely used in rubber, nevertheless, because of their low cost, and because they can be used at very high loadings with little loss of compound softness, elongation or resilience. Dry-ground limestone is probably the least expensive compounding material available and more can be loaded into rubber than any other filler. Water-ground limestone is somewhat more expensive, but offers better uniformity and finer particles size.

The much smaller size of precipitated calcium carbonates provides a corresponding increase in surface area. The ultrafine PCC products (<100 nm) can provide surface areas equivalent to the hard clays. Manipulation of manufacturing conditions allows the production of precipitated calcium carbonates of two distinct particle shapes.

The surface coating controls moisture absorption, improves dispersion, promotes better elastomer-particle contact, and protects the calcium carbonate from decomposition by acidic ingredients. There is, however, an ultrafine PCC with a reactive surface coating of chemically bonded carboxylated polybutadiene polymer. With either sulfur or peroxide vulcanization, crosslinking occurs between the elastomer and the carboxylated polybutadiene coating. This provides the highest level of reinforcement among the PCC products, comparable to thermal blacks.

Precipitated Silica – Precipitated silica is an amorphous form of silicon dioxide produced by reacting sodium silicate solution with either sulfuric acid or a mixture of carbon dioxide and hydrochloric acid. The discreet silica particles which initially form the primary particles fuse into aggregates, which in turn form loose agglomerates. The precipitate is filtered, washed of residual sodium sulfate or sodium chloride, dried, and milled. Like the carbon blacks, the precipitated silicas used in rubber are bought as agglomerates, which after milling exist in the elastomer as aggregates.

Despite similarities in size and structure between precipitated silicas and carbon blacks, fundamental differences in surface activity exist. The silica surface is highly polar and hydrophilic and contains adsorbed water. The surface hydroxyl groups are acidic and tend to retard cure rate. The adsorbed moisture volatilizes at compounding temperatures. High molecular weight polyethylene glycols are the most common additives used with precipitated silicas to reduce their reactivity toward zinc oxide and organic accelerators, and to reduce their polarity.

The use of additives to make the surface of precipitated silica less hydrophilic and more “rubberphilic” facilitates incorporation, dispersion, and more intimate filler-elastomer contact during compounding. This provides an improvement in rubber physical properties, as would be expected from a high surface area filler. However, reinforcement comparable to that obtained with carbon black requires a polymer-filler bonding mechanism comparable to that provided at the carbon black active sites. With precipitated silicas, this comes by way of a reactive silane.

Miscellaneous Fillers – Although kaolin clay, calcium carbonate, and precipitated silica account for most of the non-black fillers used in rubber today, there are a number of other fillers routinely used for their low cost or unique functionality.

Talc – Although widely used as a reinforcing filler in plastics, relatively little talc is used for this purpose in rubber. Platy talcs are white, hydrophobic, and alkaline, with greater anisometry than kaolin clay They are readily treated with silanes and other coupling agents. Micronized talcs with median particle size of 1 to 2 microns and essentially all particles <10 microns are available and are used, although they compete with the generally less expensive clays.

Barite – Barite, ground natural barium sulfate, is used in acid resistant compounds because of its inertness, and as a high gravity filler where weight is desired. It has little effect on cure, hardness, stiffness, or aging. Precipitated barium sulfate, also known as blanc fixe, is available in fine enough particle size to be semi-reinforcing. It provides the same softness and resilience as barite but better tensile strength and tear resistance.

Dialomite – As silica, dialomite is chemically inert, but its high adsorptive capacity for accelerators can affect cure. It imparts stiffness, hardness and low die swell. Dialomite is used as a filler in silicone rubber, and because of its adsorptive capacity, as a process aid in high oil rubber compounds.

Mica – Because of its platey nature, mica is occasionally used as a filler or semi-reinforcer, depending upon particle size.

Fumed Silica – Fumed silica is generally finer in primary particle size and higher in surface area than precipitated silica. As a reinforcing agent, it provides an aggregate structure similar to that of carbon black and precipitated silica. it has lower moisture content and fewer surface hydroxyls than precipitated silica.

Precipitated Silicates – These products are coarser and less structured than the precipitated silicas and, as such, are only semi-reinforcing, but can be used at high loadings. Precipitated calcium silicate and precipitated sodium aluminum silicate are the most common alternatives.