Boolean algebra:

Boolean algebra is algebra for the manipulation of objects that can take on only two values, typically true and false, although it can be any pair of values. Because computers are built as collections of switches that are either “on” or “off,” Boolean algebra is a very natural way to represent digital information.

In reality, digital circuits use low and high voltages, but for our level of understanding, 0 and 1 will suffice.

It is common to interpret the digital value 0 as false and the digital value 1 as true. In addition to binary objects, Boolean algebra also has operations that can be performed on these objects, or variables.

Combining the variables and operators yields Boolean expressions. A Boolean function typically has one or more input values and yields a result, based on these input values, in the range {0, 1}. Three common Boolean operators are AND, OR, and NOT.

OR Gate:

The Gate is a logic circuit with two or more inputs and one output. The output signal occurs only for certain combinations of input signals.

The figure below shows the OR gate symbols with two inputs (A, B) and the output (Y)

The Boolean expression for this gate is (A+B=Y); for this gate we have four cases to analyze:

Case1: A=0 &B=0   then   Y=0

Case2: A=0 &B=1   then   Y=1

Case3: A=1 &B=0   then   Y=1

Case4: A=1 &B=1   then   Y=1

All these cases put in the table to represent the truth table for the gate; so the truth table for the OR gate with two inputs become:

| A | B | Y=A+B |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 1 | 1 |

Note:  No matter how many inputs there are, the OR gate has HIGH output when any or all of the inputs are HIGH.

Most digital circuits use diodes and transistors as switch to change from one voltage level to another.

AND Gate:

The AND gate is another basic kind of digital circuits, the figure below shows the OR gate symbols with two inputs (A, B) and the output is (Y)

The Boolean expression for this gate is (A . B=Y); for this gate we have four cases to analyze:

Case1: A=0 &B=0   then   Y=0

Case2: A=0 &B=1   then   Y=0

Case3: A=1 &B=0   then   Y=0

Case4: A=1 &B=1   then   Y=1

All these cases put in the table to represent the truth table for the gate; so the truth table for the OR gate with two inputs become:

| A | B | Y=A . B |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 1 | 1 |

Note:  No matter how many inputs there are, the AND gate has HIGH output only when all of the inputs are HIGH.

Positive & Negative logic systems:

A positive logic system is a system in which a (1) is represents the more positive of two voltage levels, in a negative logic system a (1) represents for the more negative voltage. For instance; suppose a digital system has voltage levels of (+5Volt) & (0 Volt) we are free to choose the definition for (0) and (1); if we say a (1) stands for (+5Volt) and (0) for (0Volt) the system becomes a positive logic system, on the other hand if we donate (1) for (0Volt) and (0) for (+5 Volt) we have a negative logic system.

The distinction between the positive and negative logic system is important because as OR gate in a positive logic system becomes AND gate in a negative logic system.

To understand why, see fig (4-8) in your text book and the truth tables (8, 9 and 10).

A digital logic system can be either positive or negative logic system ; it al depends on how we define the (0) and(1), for consistency we shall use positive logic (1) always will stand for the more positive of the two voltage levels.

The NOT circuit:

Another type of the basic digital logic circuits is the NOT circuit, also called the (complementary) circuit or an (inverter).

This circuit has only one input and one output; all it does is invert the input, if the input is HIGH the output is LOW and vise versa

The figure below shows the NOT circuit symbols:

We call this circuit NOT because the output is not the same as the input.

The truth table of this circuit is:

| A | Y=A` |
| --- | --- |
| 0 | 0 |
| 1 | 0 |

OR addition:

In Boolean algebra when we solve an equation we get either (0) or (1). No other answers are possible because the set of numbers includes only the binary digits (0) and (1). Another deference of Boolean algebra is the meaning of the plus sign, in Boolean algebra the (+) means the action of the OR gate .

So the four cases for this gate become:

Case1:                      Y = A + B = 0 + 0 = 0

Case2:                      Y = A + B = 1 + 0 = 1

Case3:                      Y = A + B = 0 + 1 = 1

Case4:                      Y = A + B = 1 + 1 = 1

Note:                                   In the decimal addition (1+1=2)

In the binary addition (1+1=10)

In the OR addition (1+1=1)

AND multiplication:

In Boolean algebra when we write

Y = A . B or Y=A B

We mean A and B are be combined in the same way that an AND gate combines A and B to give a Y output.

So the four cases for this gate become:

Case1:                      Y = A . B = 0 . 0 = 0

Case2:                      Y = A . B = 1 . 0 = 0

Case3:                      Y = A . B = 0 . 1 = 0

Case4:                      Y = A . B = 1 . 1 = 1

The result of AND multiplication are the same as for ordinary multiplication.

Example: what is the Boolean expression for the output of the circuit shown below?

Solution:      Y = AB + B

Examples: Evaluate the Boolean expression in the preceding example for:

- A=0   and  B=1                 b)  A=1  and   B=0

Example: what is the Boolean expression for (A=0   and B=1) of the circuit below?

Solution:      Y = (A+B) B        then   Y= (0+1)1 = 1 . 1= 1

NOT operation:

In Boolean algebra (Y = A`) means you are to change A in the same way that NOT circuit does, we read this expression as Y equal NOT A.

The bar over A means the complement of the quantity, in other words

When A= 0 and Y=A` =0`=1

The NOT operation is also called (negation) or (inverse) and the prime is sometimes used instead of a bar to signify the NOT operation.

Example: what is the Boolean expression for the output of the circuit shown below?