Computer Architecture

33d  stage

20-2-2012

College of sciences for women ((())) Dept. of computer sciences

Year 2011-2012

Lec. (12)

7.4 RISC versus CISC

Many of the trade-offs presented so far have been a matter of the length and complexity of programs, and hence performance in a rough sense, versus simplicity in design and implementation.

RISC Put simply, RISC designs attempt to provide an ISA where instructions all do very simple operations. To write a program using such an ISA might take more instructions since they are so simple, but this same simplicity means a processor implementation can execute them very quickly; each instruction typically takes one clock cycle. RISC designs are characterized by the provision of many general-purpose registers, few addressing modes and a fixed length, regular instruction  encoding. This makes it easy to program for both humans and compilers; performance of compiler-generated code can often match or better that of human-generated code. 200 5 Basic Processor Design

CISC In contrast, CISC designs offer instructions which perform much more involved operations. For example, a CISC ISA might include an instruction for computing square roots while a RISC design would demand you constructed an algorithm in software using only the basic instructions available. Although the CISC instruction set will be harder to implement and instructions might take longer to execute, a program will typically be shorter since there is more work being done by each instruction. Instructions might take many clock cycles to

complete. A CISC design will typically have fewer general-purpose registers (perhaps using some of them for special-purposes), many addressing modes and might use a variable length instruction encoding. As a result, it will be harder for a compiler to generate efficient code.

7.5 Moore’s Law

Gordon Moore, co-founder of Intel, is credited with identifying an important trend with respect to development of transistor-based technology: roughly speaking, Moore’s Law says that the number of transistors that one can fit in unit area roughly doubles every two years. This has become a form of a self-fulfilling prophecy however in the sense that that the “law” is now an accepted truth so industry is forced to deliver improvements: industry is driven by the law rather than the other way around ! Figure 15 demonstrates the manifestation of Moore’s Law on the development of Intel processors. Of course Moore’s Law is not a “law” at all but has held for the last fourty or so years all the same. The implications for design of a processor can be viewed in (at least) two ways:

• If one can fit more transistors in unit area, the transistors are getting smaller and hence working faster due to their physical characteristics. As a result, one can take a fixed processor design and over time it will get faster or use less power as a result of Moore’s Law.

• If one can fit more transistors in unit area, then one can design and implement more complex structures in the same fixed area. As a result, over time one can use the extra transistors to improve the processor design yet keep it roughly the same size.

The second option above is clearly more reasonable in the long term. That is, since there are physical limits that dictate how small and fast a transistor can be, better design is always the best long-term option. However, even in this case Moore’s Law can have an impact on thinking. For example, since the development of a new processor takes many years, Moore’s Law can invalidate a given design quite quickly: what is the best design one day might no longer be the best with respect to a new generation of improved technology two years later. Furthermore, not all components obey Moore’s Law. In particular, the rate at which memory access speed has improved does not match the rate at which memory density has improved; this leads to a widening disparity between processor and memory speed which can be a significant problem.

Figure 15 A timeline of Intel processor innovation demonstrating Moore’s Law

7.6 The Control Unit

The control unit computes the control signals based on the opcode and funct fields of the instruction. The control unit produces multiplexer select, register enable, and memory write signals to control the operation of the datapath. To understand how to connect the fields of an instruction to the datapath, it is useful to review the formats of the three instruction classes:

- R-type arithmetic/logic instructions: add, sub, and, or, slt

- Memory instructions: lw, sw

- Branches: beq

There are several major observations about this instruction format that we will rely on:

■ The op field, also called the opcode, is always contained in bits 31:26. We will refer to this field as Op[5:0].

■ The two registers to be read are always specified by the rs and rt fields, at positions 25:21 and 20:16. This is true for the R-type instructions, branch equal, and for store.

■ The base register for load and store instructions is always in bit positions 25:21 (rs).

■ The 16-bit offset for branch equal, load, and store is always in positions 15:0.

■ The destination register is in one of two places. For a load it is in bit positions 20:16 (rt), while for an R-type instruction it is in bit positions 15:11 (rd). Thus we will need to add a multiplexor to select which field of the instruction is used to indicate the register number to be written.

Using this information, we can add the instruction labels and extra multiplexor (for the Write register number input of the register file) to the simple datapath. Figure 16 shows these additions plus the ALU control block, the write signals for state elements, the read signal for the data memory, and the control signals for the multiplexors. Since all the multiplexors have two inputs, they each require a single control line.

FIGURE 16 The datapath with all necessary multiplexors and all control lines identified. The control lines are shown in color. The ALU control block has also been added. The PC does not require a write control, since it is written once at the end of every clock cycle; the branch control logic determines whether it is written with the incremented PC or the branch target address.

Figure 16 shows seven single-bit control lines plus the 2-bit ALUOp control signal. Figure 17 describes the function of these seven control lines.

Now that we have looked at the function of each of the control signals, we can look at how to set them. The control unit can set all but one of the control signals based solely on the opcode field of the instruction. The PCSrc control line is the exception. That control line should be set if the instruction is branch on equal (a decision that the control unit can make) and the Zero output of the ALU, which is used for equality comparison, is true. To generate the PCSrc signal, we will need to AND together a signal from the control unit, which we call Branch, with the Zero signal out of the ALU. These nine control signals (seven from Figure 17 and two for ALUOp) can now be set on the basis of six input signals to the control unit, which are the opcode bits. Figure 18 shows the datapath with the control unit and the control signals. Figure 19 defines how the control signals should be set for each opcode;

FIGURE 17 The effect of each of the seven control signals. When the 1-bit control to a two way multiplexor is asserted, the multiplexor selects the input corresponding to 1. Otherwise, if the control is deasserted, the multiplexor selects the 0 input.

FIGURE 18 The simple datapath with the control unit. The input to the control unit is the 6-bit opcode field from the instruction. The outputs of the control unit consist of three 1-bit signals that are used to control multiplexors (RegDst, ALUSrc, and MemtoReg), three signals for controlling reads and writes in the register file and data memory (RegWrite, MemRead, and MemWrite), a 1-bit signal used in determining whether to possibly branch (Branch), and a 2-bit control signal for the ALU (ALUOp). An AND gate is used to combine the branch control signal and the Zero

output from the ALU; the AND gate output controls the selection of the next PC. Notice that PCSrc is now a derived signal, rather than one coming directly from the control unit. Thus we drop the signal name in subsequent figures.

7.6.1 Operation of the Datapath

With the information contained in Figures 17 and 19, we can design the control unit logic, but before we do that, let’s look at how each instruction uses the datapath. In the next few figures, we show the flow of three different instruction classes through the datapath. The asserted control signals and active datapath elements are highlighted in each of these. Note that a multiplexor whose control is 0 has a definite action, even if its control line is not highlighted. Multiple-bit control signals are highlighted if any constituent signal is asserted.

FIGURE 19 The setting of the control lines is completely determined by the opcode fields of the instruction. The first row of the table corresponds to the R-format instructions (add, sub, and, or, and slt). For all these instructions, the source register fields are rs and rt, and the destination register field is rd; this defines how the signals ALUSrc and RegDst are set. Furthermore, an R-type instruction writes a register (RegWrite = 1), but neither reads nor writes data memory. When the Branch control signal is 0, the PC is unconditionally replaced with PC + 4; otherwise, the PC is replaced by the branch target if the Zero output of the ALU is also high. The ALUOp field for R-type instructions is set to 10 to indicate that the ALU control should be generated from the funct field. The second and third rows of this table give the control signal settings for lw and sw. These ALUSrc and ALUOp fields are set to perform the address calculation. The MemRead and MemWrite are set to perform the memory access. Finally, RegDst and RegWrite are set for a load to cause the result to be stored into the rt register. The branch instruction is similar to an R-format operation, since it sends the rs and rt registers to the ALU. The ALUOp field for branch is set for a subtract (ALU control = 01), which is used to test for equality. Notice that the MemtoReg field is irrelevant when the RegWrite signal is 0: since the register is not being written, the value of the data on the register data write port is not used. Thus, the entry MemtoReg in the last two rows of the table is replaced with X for don’t care. Don’t cares can also be added to RegDst when RegWrite is 0. This type of don’t care must be added by the designer, since it depends on knowledge of how the datapath works.

Figure 20 shows the operation of the datapath for an R-type instruction, such as add $t1,$t2,$t3. Although everything occurs in 1 clock cycle, we can think of four steps to execute the instruction; these steps are ordered by the flow of information:

1. The instruction is fetched, and the PC is incremented.

2. Two registers, $t2 and $t3, are read from the register file, and the main control unit computes the setting of the control lines during this step also.

3. The ALU operates on the data read from the register file, using the function code (bits 5:0, which is the funct field, of the instruction) to generate the ALU function.

4. The result from the ALU is written into the register file using bits 15:11 of the instruction to select the destination register ($t1).

Similarly, we can illustrate the execution of a load word, such as lw $t1, offset($t2)  in a style similar to Figure 20. Figure 21 shows the active functional units and asserted control lines for a load. We can think of a load instruction as operating in five steps (similar to the R-type executed in four):

1. An instruction is fetched from the instruction memory, and the PC is incremented.

2. A register ($t2) value is read from the register file.

3. The ALU computes the sum of the value read from the register file and the sign-extended, lower 16 bits of the instruction (offset).

4. The sum from the ALU is used as the address for the data memory.

FIGURE 20 The datapath in operation for an R-type instruction such as add $t1,$t2,$t3. The control lines, datapath units, and connections that are active are highlighted.

5. The data from the memory unit is written into the register file; the register destination is given by bits 20:16 of the instruction ($t1) .

FIGURE 21 The datapath in operation for a load instruction. The control lines, datapath units, and connections that are active are highlighted. A store instruction would operate very similarly. The main difference would be that the memory control would indicate a write rather than a read, the second register value read would be used for the data to store, and the operation of writing the data memory value to the register file would not occur.

Finally, we can show the operation of the branch-on-equal instruction, such as beq $t1,$t2,offset, in the same fashion. It operates much like an R-format instruction, but the ALU output is used to determine whether the PC is written with PC + 4 or the branch target address. Figure 22 shows the four steps in execution:

1. An instruction is fetched from the instruction memory, and the PC is incremented.

2. Two registers, $t1 and $t2, are read from the register file.

3. The ALU performs a subtract on the data values read from the register file. The value of PC + 4 is added to the sign-extended, lower 16 bits of theinstruction (offset) shifted left by two; the result is the branch target address.

FIGURE 22 The datapath in operation for a branch equal instruction. The control lines, datapath units, and connections that are active are highlighted. After using the register file and ALU to perform the compare, the Zero output is used to select the next program counter from between the two candidates.

4. The Zero result from the ALU is used to decide which adder result to store into the PC.