Lec. (9)

Microprocessors

2nd  stage

23-2-2012

College of sciences for women ((())) Dept. of computer sciences

Year 2011-2012

12.2  ADDRESSING MODES.

The different ways in which operands can be addressed are called the addressing modes. Addressing modes differ in the way the address information of operands is specified. The simplest addressing mode is to include the operand itself in the instruction, that is, no address information is needed. This is called immediate addressing. A more involved addressing mode is to compute the address of the operand by adding a constant value to the content of a register. This is called indexed addressing. Between these two addressing modes there exist a number of other addressing modes including absolute addressing, direct addressing, and indirect addressing. A number of different addressing modes are explained below.

12.2.1  Immediate Mode

According to this addressing mode, the value of the operand is (immediately) available in the instruction itself. Consider, for example, the case of loading the decimal value 1000 into a register Ri. This operation can be performed using an instruction such as the following: LOAD #1000, Ri. In this instruction, the operation to be performed is to load a value into a register. The source operand is (immediately) given as 1000, and the destination is the register Ri. It should be noted that in order to indicate that the value 1000 mentioned in the instruction is the operand itself and not its address (immediate mode). As can be seen the use of the immediate addressing mode is simple. The use of immediate addressing leads to poor programming practice. This is because a change in the value of an operand requires a change in every instruction that uses the immediate value of such an operand. A more flexible addressing mode is explained below.

12.2.2  Direct (Absolute) Mode

According to this addressing mode, the address of the memory location that holds the operand is included in the instruction. Consider, for example, the case of loading the value of the operand stored in memory location 1000 into register Ri. This operation can be performed using an instruction such as LOAD 1000, Ri. In this instruction, the source operand is the value stored in the memory location whose address is 1000, and the destination is the register Ri. Note that the value 1000 is not prefixed with any special characters, indicating that it is the (direct or absolute) address of the source operand. Figure 13 shows an illustration of the direct addressing mode.

Figure 13 Illustration of the direct addressing mode

For example, if the content of the memory location whose address is 1000 was (2345) at the time when the instruction LOAD 1000, Ri is executed, then the result of executing such instruction is to load the value (2345) into register Ri. Direct (absolute) addressing mode provides more flexibility compared to the immediate mode. A more flexible addressing mechanism is provided through the use of the indirect addressing mode. This is explained below.

12.2.3  Indirect Mode

In the indirect mode, what is included in the instruction is not the address of the operand, but rather a name of a register or a memory location that holds the (effective) address of the operand. In order to indicate the use of indirection in the instruction, it is customary to include the name of the register or the memory location in parentheses. Consider, for example, the instruction LOAD (1000), Ri. This instruction has the memory location 1000 enclosed in parentheses, thus indicating indirection. The meaning of this instruction is to load register Ri with the contents of the memory location whose address is stored at memory address 1000. Because indirection can be made through either a register or a memory location, therefore, we can identify two types of indirect addressing. These are register indirect addressing, if a register is used to hold the address of the operand, and memory indirect addressing, if a memory location is used to hold the address of the operand. The two types are illustrated in Figure 14.

Figure 14 Illustration of the indirect addressing mode

12.2.4  Indexed Mode

In this addressing mode, the address of the operand is obtained by adding a constant to the content of a register, called the index register. Consider, for example, the instruction LOAD X(Rind), Ri. This instruction loads register Ri with the contents of the memory location whose address is the sum of the contents of register Rind and the value X. Index addressing is indicated in the instruction by including the name of the index register in parentheses and using the symbol X to indicate the constant to be added. Figure 15 illustrates indexed addressing. As can be seen, indexing requires an additional level of complexity over register indirect addressing.

Figure 15 Illustration of the indexed addressing mode

12.2.5. Other Modes

The addressing modes presented above represent the most commonly used modes in most processors. They provide the programmer with sufficient means to handle most general programming tasks. However, a number of other addressing modes have been used in a number of processors to facilitate execution of specific programming tasks. These additional addressing modes are more involved as compared to those presented above. Among these addressing modes the relative, autoincrement, and the autodecrement modes represent the most well-known ones. These are explained below.

1-Relative Mode

Recall that in indexed addressing, an index register is used. Relative addressing is the same as indexed addressing except that the program counter (PC) replaces the index register. For example, the instruction LOAD X(PC), Ri  loads register Ri with the contents of the memory location whose address is the sum of the contents of the program counter (PC) and the value X. Figure 16 illustrates the relative addressing mode.

2-Autoincrement Mode

This addressing mode is similar to the register indirect addressing mode in the sense that the effective address of the operand is the content of a register, call it the autoincrement register, that is included in the instruction.

Figure 16 Illustration of relative addressing mode

However, with autoincrement, the content of the autoincrement register is automatically incremented after accessing the operand. As before, indirection is indicated by including the autoincrement register in parentheses. The automatic increment of the register’s content after accessing the operand is indicated by including a (‏) after the parentheses. Consider, for example, the instruction LOAD (Rauto)‏, Ri. This instruction loads register Ri with the operand whose address is the content of register Rauto. After loading the operand into register Ri, the content of register Rauto is incremented, pointing for example to the next item in a list of items. Figure 17 illustrates the autoincrement addressing mode.

Figure 17 Illustration of the autoincrement addressing mode

3-Autodecrement Mode

Similar to the autoincrement, the autodecrement mode uses a register to hold the address of the operand. However, in this case the content of the autodecrement register is first decremented and the new content is used as the effective address of the operand. In order to reflect the fact that the content of the autodecrement register is decremented before accessing the operand, a (ــــ) is included before the indirection parentheses. Consider, for example, the instruction LOAD ــــ (Rauto), Ri. This instruction decrements the content of the register Rauto and then uses the new content as the effective address of the operand that is to be loaded into register Ri. Figure 18 illustrates the autodecrement addressing mode.

Figure 18 Illustration of the autodecrement addressing mode

The seven addressing modes presented above are summarized in Table 2. In each case, the table shows the name of the addressing mode, its definition, and a generic example illustrating the use of such mode.

TABLE 2 Summary of Addressing Modes