Laser Safety Quiz

Name: ___________________________________ Date: ______________

Employee No: ____________________________  Union:______________

Supervisor: __________________________ Dept.: __________________

- Power emitted from a Class II laser does not exceed which of the following?

  - 0.5 milliwatt

  - 1 milliwatt

  - 5 milliwatts

  - 0.1 milliwatt

- The visible portion of the electromagnetic spectrum is generally defined as which of the following wavelength ranges?

  - 100 nm to 400 nm.

  - 400 nm to 700 nm.

  - 1 micron to 1.5 microns.

  - 1040 nm to 10600 nm.

- Which of the following is NOT an example of invisible laser radiation?

  - A 10,600 nm CO2 laser beam.

  - A 1064 nm Nd:YAG laser beam.

  - Ultraviolet radiation.

  - A 532 nm Nd:YAG laser beam.

- Solids, liquids, and gases are examples of which of the following?

  - An excitation system.

  - An active medium.

  - An optical resonator.

  - A laser property.

- If must enter the Nominal Hazard Zone, what is the best thing you can do to protect yourself from potential hazardous exposure?

  - Notify the LSO.

  - Activate the warning system.

  - Wear personal protective equipment.

  - Stand near the temporary barrier.

- Which class(s) of laser would be considered safe if not viewed longer than the eye’s aversion response?

  - Class II

  - Class IIIa

  - Class IIIb

  - Class IV

- Hazardous exposure to diffusely reflected radiation is most likely to occur from which class of laser?

  - Class II.

  - Class IIIa

  - Class IIIb

  - Class IV

- Which classes of lasers require the word “DANGER” on their warning labels?

  - Classes I,II, and IIa.

  - Classes II and IIa.

  - Classes IIIb, and IV and some IIIa.

  - Classes IIIb and IV only.

- Which of the following is not a property of laser radiation?

  - Monochromacity

  - Coherence

  - Directionality

  - Low Radiance

- What is last task you should perform after you have finished servicing the laser?

  - Put away any tools.

  - Inform the LSO.

  - Reactivate all the safety features.

  - Look directly into the beam.

- Which type of reflection is caused when a beam strikes a mirror-like surface?

  - Direct

  - Intrabeam

  - Diffuse

  - Specular

- Which of the following reason is why a light bulb is not a hazardous source of light?

  - Its waves radiate in all directions.

  - Its waves radiate in one direction.

  - It is monochromatic.

  - It has a very high irradiance.

- Which of the following activities creates airborne contaminants?

  - Dyes being dissolved in a solvent.

  - Large amounts of inert buffer gasses released into the air.

  - A high-powered laser beam interacting with metal.

  - All of the above.

- Which of the following is the transparent outer surface of the eye?

  - Retina

  - Lens

  - Cornea

  - Fovea

- The acronym L.A.S.E.R. stands for which of the following terms?

  - Light Amplification by Stimulated Emission of Radiation.

  - Light Amplification by Spontaneous Emission of Radiation.

  - Light Alteration by Stimulated Emission of Radiation.

  - Light Alteration by Spontaneous Energy of Radiation.

- Which type of eye damage is likely to occur if the lens is exposed to ultraviolet laser radiation?

  - Cataract

  - Retinal Burn

  - Stigmatism

  - Total Blindness

- Which is one of the most important pieces of information included in the Standard Operating Procedure for a laser?

  - Identification of potential hazards.

  - The name of the LSO.

  - The laser manufacturer.

  - Medical requirements.

- What does the Nominal Hazard Zone (NHZ) represent?

  - The focal point of a converging beam.

  - The time it takes to reach safe exposure levels.

  - The region within which the level of the laser radiation exceeds the MPE.

  - The region from within which it is safe to view the beam.

- What is the best way to control airborne contaminants?

  - Set up a temporary controlled area.

  - Use flame retardant curtains.

  - Use an exhaust ventilation system

  - Do not use target materials.

- Which of the following laser wavelengths can cause retinal burns?

  - Far infrared and ultraviolet.

  - Visible and near infrared.

  - Ultraviolet only.

  - Far infrared only.

- Which one of the following is a device intended to prevent access to hazardous laser radiation when an enclosure is opened?

  - Protective housing

  - Warning system

  - Interlock

  - Beam stop

- Which one of the following is NOT a type of collateral radiation normally associated with laser output?

  - A laser beam

  - X-ray

  - Radio frequency

  - Ultraviolet

To complete the refresher training course, please send this completed test to:

Richard Grundsten

Department of Occupational Health and Safety

East Office Building, Room C37

DOHS will inform you of the test results.