Lec. (2)

Computer Organization

First stage

College of Computer Technology

Dept.Information Networks

Characters and strings of text:

A personal computer processes textual information. Individual key-strokes generate letters, numbers, or other symbols that are called characters. Groups of characters treated as  units are called strings. Since characters and strings can’t be processed directly by machine that “understand” only binary numbers, they must be encoded in some kind of binary code. The computer uses coding system called ASCII (American Standard Code for Information Interchange). ASCII associates a unique 8-bits binary with each character symbol.

The standard ASCII uses 7 bits to encode a character. Thus, 27=128 different characters can be represented. This number is sufficiently large to represent uppercase and lowercase characters, digits, special characters such as !,ˆ, and control characters such as CR (carriage return), LF (line feed), and so on.

Since we store the bits in units of a power of 2, we end up storing 8 bits for each character, even though ASCII requires only 7 bits. The eighth bit is put to use for two purposes:

- To Parity Encode for Error Detection: The eighth bit can be used to represent the parity bit. This bit is made 0 or 1 such that the total number of 1s in a byte is even (for even parity) or odd (for odd parity). This can be used to detect simple errors in data transmission.

- To Represent an Additional 128 Characters: By using all 8 bits we can represent a total of 28=256 different characters. This is referred to as extended ASCII.

Thus, an additional 128 encodings have been added, mostly to take care of the Latin letters, accents, and diacritical marks. . The biggest decimal equivalent we can express in 8-bits is 11111111, which is the sum of all powers of two from zero to seven.

11111111 binary = 128 + 64 + 32 + 16 + 8 + 4 + 2 + 1

= 255 decimal.

In ASCII the letter A is associated with 01000001 binary. The numeral (not its value) 1 is associated with 00110001 binary. People usually convert binary codes to their decimal equivalents when they refer to them. Hence, A is represented by the decimal number 65 and numeral 1 is represented by decimal number 49.

Suppose you want to store a line in computer memory. You enter the string of characters into the computer through the keyboard. The keyboard converts each key-stroke into ASCII binary code.

Everything is exactingly stored as binary numbers: letters of alphabet, numerals, punctuation marks, and special characters ($, #, %, and so on). Each character has its own 8-bits code. The memory of computer can record binary digits and nothing else, so all kinds of information must be encoded. The power of a computer to manipulate symbols is hidden in the simplicity of coding scheme.

Table 2: illustrates hexa and decimal codes for ASCII control codes

Table 3: illustrates hexa and decimal codes for ASCII printable character codes.

HW// Convert the following binary numbers to its decimal codes :

00110011

00000111

10000001

10101010

11110000