1.1 3. Color Image

Color image can be modeled as three band monochrome image data, where each band of the data corresponds to a different color.

Figure (1.8) Color Images.

The actual information stored in the digital image data is brightness information in each spectral band. When the image is displayed, the corresponding brightness information is displayed on the screen by picture elements that emit light energy corresponding to that particular color.

Typical color images are represented as red, green, and blue or RGB images .using the 8-bit monochrome standard as a model, the corresponding color image would have 24 bit/pixel – 8 bit for each color bands (red, green and blue). The following figure we see a representation of a typical RGB color image.

Figure (1.9) Typical RGB color image can be thought as three

separate images IR(r,c),IG(r,c),IB(r,c) [1]

The following figure illustrate that in addition to referring to arrow or column as a vector, we can refer to a single pixel red ,green, and blue values as a color pixel vector –(R,G,B ).

Figure (1.10) A color pixel vector consists of the red, green and blue pixel values  (R, G, B) at one given row/column pixel coordinate( r , c) [1].

For many applications, RGB color information is transformed into mathematical space that that decouples the brightness information from the color information.

The hue/saturation /lightness (HSL) color transform allows us to describe colors in terms that we can more readily understand.

Figure (1.11) HSL Color Space [1].

The lightness is the brightness of the color, and the hue is what we normally think of as “color” and the hue (ex: green, blue, red, and orange).

The saturation is a measure of how much white is in the color (ex: Pink is red with more white, so it is less saturated than a pure red).

[Most people relate to this method for describing color}.

Example: “a deep, bright orange” would have a large intensity (“bright”), a hue of “orange”, and a high value of saturation (“deep”).we can picture this color in our minds, but if we defined this color in terms of its RGB components, R=245, G=110 and B=20.

Modeling the color information creates a more people oriented way of describing the colors.

4. Multispectral Images

Multispecrtal images typically contain information outside the normal human perceptual range. This may include infrared (تحت الحمراء), ultraviolet (فوق البنفسجية), X-ray, acoustic or radar data. Source of these types of image include satellite systems underwater sonar systems and medical diagnostics imaging systems.

1.10 Digital Image File Format

Why do we need so many different types of image file format?

• The short answer is that there are many different types of images

and application with varying requirements.

• A more complete answer, also considers market share proprietary

information, and a lack of coordination within the imaging industry.

Many image types can be converted to one of other type by easily available image conversion software. Field related to computer imaging is that computer graphics.

Computer Graphics:

Computer graphics is a specialized field within that refers to the computer science realm that refers to the reproduction of visual data through the use of computer.

In computer graphics, types of image data are divided into two primarily categories:

1.  Bitmap image (or raster image): can represented by our image model       I(r, c), where we have pixel data and corresponding brightness values stored in file format.

2.  Vector images: refer to the methods of representing lines, curves shapes by storing only the key points. These key points are sufficient to define the shapes, and the process of turing theses into an image is called rending after the image has been rendered, it can be thought of as being in bit map format where each pixel has specific values associated with it.

Most the type of file format fall into category of bitmap images. In general, these types of images contain both header information and the raw pixel data. The header information contain information regarding

1.  The number of rows (height)

2.  The number of columns (Width)

3.  The number of bands.

4.  The number of bit per pixel.

5.  The file type

6.  Additionally, with some of the more complex file formats, the header may contain information about the type of compression used and other necessary parameters to create the image, I(r,c).

Image File Format:

1.  BMP format:

It is the format used by the windows, it’s a compressed format and the data of image are located in the field of data while there are two fields, one for header (54 byte)  that contains the image information such as (height ,width , no. of bits per pixel, no of bands , the file type).

The second field is the color map or color palette for gray level image, where its length is 0-255).

2.  Bin file format:

It is the raw image data I(r,c) with no header information.

3. PPM file format:

It contain raw image data with simplest header, the PPM format, include PBM(binary),PGM(gray),PPM (color), the header contain a magic number  that identifies the file.

4.  TIFF (Tagged Image File Format) and GIF(Graphics Interchange

Format):

They are used on World Wide Web (WWW). GIF files are limited to a maximum of 8 bits/pixel and allows for a type of compression called LZW. The GIF image header is 13 byte long & contains basic information.

5.  JPEG (Joint photo Graphic Experts Group):

It is simply becoming standard that allows images compressed algorithms to be used in many different computer platforms. JPEG images compression is being used extensively on the WWW. It’s, flexible, so it can create large files with excellent image equality.

6.  VIP(visualization in image processing )formats:

It is developed for the CVIP tools software, when performing temporary images are created that use floating point representation which is beyond the standard 8 bit/pixel. To represent this type of data the remapping is used, which is the process of taking original image and adding an equation to translate it to the rang (0-225).

Image Analysis

2.1 Image Analysis

Image analysis involves manipulating the image data to determine exactly the information necessary to help solve a computer imaging problem. This analysis is typically part of a larger process, is iterative in nature and allows us to answer application specific equations: Do we need color information? Do we need to transform the image data into the frequency domain? Do we need to segment the image to find object information? What are the important features of the image? Image analysis is primarily data reduction process. As we have seen, images contain enormous amount of data, typically on the order hundreds of kilobytes or even megabytes. Often much of this information is not necessary to solve a specific computer imaging problem, so primary part of the image analysis task is to determine exactly what information is necessary. Image analysis is used both computer vision and image processing.

For computer vision, the end product is typically the extraction of high-level information for computer analysis or manipulation. This high-level information may include shape parameter to control a robotics manipulator or color and texture features to help in diagnosis of a skin tumor.

In image processing application, image analysis methods may be used to help determine the type of processing required and the specific parameters needed for that processing. For example, determine the degradation function for an image restoration procedure, developing an enhancement algorithm and determining exactly what information is visually important for image compression methods.

2.2 System Model

The image analysis process can be broken down into three primary stages:

1.  Preprocessing.

2.  Data Reduction.

3.  Features Analysis.

1. Preprocessing:

Is used to remove noise and eliminate irrelevant, visually unnecessary information. Noise is unwanted information that can result from the image acquisition process, other preprocessing steps might include:

• Gray –level or spatial quantization (reducing the number of bits per pixel or the image size).

• Finding regions of interest for further processing.

2. Data Reduction:

Involves either reducing the data in the spatial domain or transforming it into another domain called the frequency domain, and then extraction features for the analysis process.

3. Features Analysis:

The features extracted by the data reduction process are examine and evaluated for their use in the application.

After preprocessing we can perform segmentation on the image in the spatial domain or convert it into the frequency domain via a mathematical transform. After these processes we may choose to filter the image. This filtering process further reduces the data and allows us to extract the feature that we may require for analysis.

2.3 Preprocessing

The preprocessing algorithm, techniques and operators are use to perform initial processing that makes the primary data reduction and analysis task easier. They include operations related to:

• Extracting regions of interest.

• Performing basic algebraic operation on image.

• Enhancing specific image features.

• Reducing data in resolution and brightness.

Preprocessing is a stage where the requirements are typically obvious and simple, such as removal of artifacts from images or eliminating of image information that is not required for the application. For example, in one application we needed to eliminate borders from the images that have been digitized from film. Another example of preprocessing step involves a robotics gripper that needs to pick and place an object ; for this we reduce a gray-level image to binary (two-valued) image that contains all the information necessary to discern the object ‘s outlines.

2.3.1 Region –of-Interest Image Geometry

Often, for image analysis we want to investigate more closely a specific area within the image, called region of interest (ROI). To do this we need operation that modifies the spatial coordinates of the image, and these are categorized as image geometry  operations. The image geometry operations discussed here include:

Crop (قص), Zoom, enlarge, shrink, translate and rotate. The image crop process is the process of selecting a small portion of the image, a sub image and cutting it away from the rest of the image. After we have cropped a sub image from the original image we can zoom in on it by enlarge it. The zoom process can be done in numerous ways:

1.  Zero-Order Hold.

2.  First _Order Hold.

3.  Convolution.

- Zero-Order hold: is performed by repeating previous pixel values, thus creating a blocky effect as in the following figure:

Figure (2.1): Zero _Order Hold Method

2.  First _Order Hold:  is performed by finding linear interpolation between a adjacent pixels, i.e., finding the average value between two pixels and use that as the pixel value between those two, we can do this for the rows first as follows:

Original Image Array                         Image with Rows Expanded

The first two pixels in the first row are averaged (8+4)/2=6, and this number is inserted between those two pixels. This is done for every pixel pair in each row.

Next, take result and expanded the columns in the same way as follows:

Image with rows and columns expanded

This method allows us to enlarge an N×N sized image to a size of              (2N-1) × (2N-1) and be repeated as desired.