Lec. (4)

Computer Organization

1st stage

College of Computer Technology

10-Computer Arithmetic: ( Integer, Fixed-point, and Floating-point arithmetic ).

A computer designer chooses methods of representation in computer on the basis of an evaluation of cost and speed considerations, and occasionally on the basis of accuracy and programmer convenience. A computer design is then chosen that has operations to handle information in those representations. Normally, just a single representation is used for character data. However, a single representation for numeric information is inadequate for a wide range of problems, so general purpose computers frequently have more than one form of number representation. Typically, there are integer and floating-point representation.

Fixed-point is one whose point (decimal or binary) is in a fixed place in relation to the digits. For example, both integers and fractions can be represented as fixed-point numbers. For integers, the point is fixed at the extreme right. For fractions, the point is fixed at the extreme left. Since the point position is fixed, it does not have to be store in memory with the value.

Floating-point Representation.

Floating-point (FP) number is one in which the position of the point can vary from number to number.

Ex1:-  125  0.125 × 103  1.25 × 102  12.5 × 101

0.125 × E3  1.25 × E2

Ex2:-  0.00127  12.7 × 10-4  1.27 × 10-3  0.0127 × 10-1

12.7 × E-4

In general form

∓M×2∓e

∓M×E∓P   or

Ex3:-  110.110 0.110110 × (2)3  0.110110 × (10)11

In digital computers, floating-point numbers consist of three parts (figure 3):

- a sign bit.

- an exponent part (representing the exponent on a power of 2) (P).

- a fractional part called the mantissa (M) part, represents a signed fixed-point number.

Figure 3 Representation of a floating-point number

1bit

Sign bit

Exponent

Mantissa

n bits

m bits

Ex4:- (+6132.789)10 can be represented in floating-point as:

+0.6132789 × 10+4

The mantissa here is considered to be a fixed-point fraction, so the decimal point is assumed to be at the left of the most significant digit. The exponent contains the decimal number +4 to indicate that the actual position of the decimal point is four decimal position to the right of the assumed decimal point.

The number of bits used for the exponent and mantissa depends on whether we would like to optimize for range (more bits in the exponent) or precision (more bits in the mantissa).

Suppose we will use a 14-bits model with a 5-bit exponent, an 8-bit mantissa, and a sign bit (see Figure 4)

Figure 4: Floating-Point Representation with 14 bits model

1bit

Sign bit

Exponent

Mantissa

5 bits

8 bits

0

00101

10001000

Let's say that we wish to store the decimal number 17 in our model. We know that 17 = 17.0 x 100 = 1.7 x 101 = 0.17 x 102. Analogously, in binary, 1710 = 100012 x 20 = 1000.12 x 21 = 100.012 x 22 = 10.0012 x 23 = 1.00012 x 24 = 0.100012 x 25. If we use this last form, our fractional part will be 10001000 and our exponent will be 00101, as shown here:

Using this form, we can store numbers of much greater magnitude than we could using a fixed-point representation of 14 bits (which uses a total of 14 binary digits). If we want to represent 65536 = 0.12 x 217 in this model, we have:

0

10001

10000000

One obvious problem with this model is that we haven't provided for negative exponents. The exponent in the above figures, can only represent positive numbers. If we wanted to store 0.25 we would have no way of doing so because 0.25 is 2 – 2 and the exponent –2 cannot be represented.  To represent both positive and negative exponents, a fixed value, called a bias, is used.

The idea behind using a bias value is to convert every integer in the range into a non-negative integer, which is then stored as a binary numeral. The integers in the desired range of exponents are first adjusted by adding this fixed bias value to each exponent. The bias value is a number near the middle of the range of possible values. In this case, we could select 16 because it is midway between 0 and 31 (our exponent has 5 bits, thus allowing for 25 or 32 values). Any number larger than 16 in the exponent field will represent a positive value. Values less than 16 will indicate negative values. This is called an excess-16 representation because we have to subtract 16 to get the true value of the exponent.

0

10101

10001000

Returning to our example of storing 17, we calculated 1710 = 0.100012 x 25. The biased exponent is now 16 + 5 = 21:

If we wanted to store 0.25 = 1.0 x 2-2 we would have:

0

01110

10000000

To simplify performing operations on FP numbers and to increase their precision, they are always represented in what is called normalized forms. A floating-point number is said to be normalized if the most significant position of the mantissa contains a non-zero digit. That is normalized in a computer consist of the left shifting the mantissa until the first digit is non-zero, while reducing the exponent correspondingly.

Ex5:-  0.000173 × 100  0.001730 × 10-1  0.017300 × 10-2 

0.173000 × 10-3 → normalized

When the mantissa is normalized, it has no leading zeros.

The Pentium supports three formats for floating-point numbers: two of these are for external use and one for internal use. The internal format is used to store temporary results, and we do not discuss this format. The remaining two formats are shown below:

Figure 5: illustrates short real and long real formats, respectively.

For short real, the bias used is 127D, and for long real is 1023D.

Therefore, among the three possible representations for 1.75 in figure 6, the first representation is normalized and should be used.

Figure 6: Different representation of an FP number

11- Floating-Point Arithmetic Addition/Subtraction.

If we wanted to add two decimal numbers that are expressed in scientific notation, such as 1.5 x 102 + 3.5 x 103, we would change one of the numbers so that both of them are expressed in the same power of the base. In our example, 1.5 x 102 + 3.5 x 103 = 0.15 x 103 + 3.5 x 103 = 3.65 x 103. Floating-point addition and subtraction work the same way, as illustrated below.

Example 5:

Add the following binary numbers as represented in a normalized 14-bit format with a bias of 16.

We see that the addend is raised to the second power. Alignment of these two operands on the binary point gives us:

Renormalizing, we retain the larger exponent and truncate the low-order bit. Thus, we have:

Multiplication and division are carried out using the same rules of exponents applied to decimal arithmetic, such as 2– 3 x 24 = 21, for example.

Ex6:- Consider a register that can accommodate mantissa of 5 decimal digits and sign. The number +0.35746 × 102 (+35.746) is normalized because the mantissa has a non-zero digit (3) in the most significant position. This number can be represented in unnormalized form as +0.00357 × 104 (+35.7). This unnormalized number contains two most significant zeros, and therefore the mantissa can accommodate only three significant digits. The two last significant digits 4 and 6, that were accommodate in the normalized form, have no room in the unnormalized form because the register can only accommodate 5 digits.