Applications of conductometric Titration:

Strong acid with weak base:

The titration of a strong acid with a moderately weak base may be illustrated by the neutalisation of dilute H2SO4 by dilute NH3.The first branch of the graph reflects the disappearance  of the hydrogen ions during the neutralization ,but after  the end point has been reached the graph becomes almost horizontial ,since the excess aqueous ammonia is not appreciably ionized in the presence of ammonia sulphate.

Weak acid with a strong base

:

In the titration of a weak  acid with a strong  base ,the shape of the curve will depend upon the concentration and the dissociation constant of the acid .Thus in the neutralization of acetic acid with sodium hydroxide.the salt (sodium acetate) which is formed during the first part of the titration tends to repress  the ionization of the acetic acid  still present so that its conductance decreases. The rising salt concentration will however tend to produce an increase in  conductance .In  consequence of these opposing influences the titration curves may have minima,the position of which will depend upon the concentration and upon the strength of the weak acid .As the titration proceeds , a some what indefinite break will occur at the end point, and the graph will become linear after all the acid has been neutralized.

Very weak acid with a strong base:

The initial conductance is very small ,but increases as the neutralization proceeds  owing to the salt formed.The conductance values near thr equivalence point are high  because  of hydrolysis is considerably  reduced by the excess alkali.To determine the end point  values the conductance removed from the equivalence point must therefore be used for extrapolation.

.