Lec. (17)

Microprocessors

2nd  stage

5-5-2011

College of sciences for women ((())) Dept. of computer sciences

Year 2010-2011

21.5  Assembly Language Programming the 8086 Architecture

In order to write an executable assembly language program for the 8086 processor that will run natively on your PC you must, at a minimum, follow the rules of MSDOS®. This requires that you do not preset the values of the code segment register because it will be up to the operating system to initialize this register value when it loads the program into memory. Thus, in the 68K environment when we want to write relocatable code, we would use the PC or address register relative addressing modes. Here, we allow the operating system to specify the initial value of the CS register.

Assemblers such as Borland’s Turbo Assembler (TASM®) and Microsoft’s MASM® assembler handle many of these housekeeping tasks for you. So, as long as you follow the rules you may still be able to write assembly language programs that are well-behaved. Certainly these programs can run on any machine that is still running the 16-bit compatible versions of the various PC operating systems. The newer, 32-bit versions are more problematic because the run older DOS programs in an emulation mode which may or may not recognize the older BIOS calls. However, most simple assembly language programs which do simple console I/O should run without difficulty in a DOS window.

Let’s first look at the issue of the segment directives and memory models. In general, it is necessary to explicitly identify the portions of your program that will deal with the code, the data and the stack. This is similar to what you’ve already seen. We use the directives:

• .code

• .stack

• .data

to denote the locations of these segments in your code (note that the directives are preceded by a period). For example, if you use the directive:

.stack 100h

you are reserving 256 bytes of stack space for this program. You do not have to specify where the stack itself is located because the operating system is managing that for you and the operating system is already up and running when it is loading this program.

The .data directive identifies the data space of your program. For example, you might have the following variables in your program:

.data

var16 dw 0AAAAh

var8 db 55h

initMsg db ‘Hello World’,0Ah,0Dh

This data space declares three variables, var16, var8 and initMsg and initializes them. In order for you to use this data space in your program you must initialize the DS segment register to address of the data segment. But since you don’t know where this is, you do it indirectly:

MOV AX,@data ;Address of data segment

MOV DS,AX

Here, @data is a reserved word that causes the assembler to calculate the correct DS segment value.

The .code directive identifies the beginning of your code segment. The CS register will initialized to point to the beginning of this segment whenever the program is loaded into memory.

In addition to identifying where in memory the various program segments will reside you need to provide the assembler (and the operating system) with some idea of the type of addressing that will be required and the amount of memory resources that your program will need. You do this with the .model directive. Specifying the model indicates the size of your program and data space requirements. The available memory models are:

• Tiny: Both program code and data fit within the same 64K segment. Also, both code and

data are defined as near, which means that they are branched to by reloading the IP register.

• Small: Program code fits entirely within a single 64K segment and the data fits entirely

within a separate 64K segment. Both code and data are near.

• Medium: Program code may be larger than 64K but program data must be small enough

to fit within a single 64K segment. Code is defined as far, which means that both segment

and offset must be specified while data accesses are all near.

• Compact: Program code fits within a single 64K segment but the size of the data may

exceed 64K, with no single data element, such as an array, being larger than 64K. Code

accesses are near and data accesses are far.

• Large: Both code and data spaces may be larger than 64K. However, no single data array

may be larger than 64K. All data and code accesses are far.

• Huge: Both code and data spaces may be larger than 64K and data arrays may be larger

than 64K. Far addressing modes are used for all code, data and array pointers.

The use of memory models is important because they are consistent with the memory models used by compilers for the PC. It guarantees that an assembly language module that will be linked in with modules written in a high level language will be compatible with each other.

Let’s examine a simple program that could run on in a DOS emulation window on your PC.

.MODEL small

.STACK 100h

.DATA

PrnStrg db ‘Hello World$’ ;String to print

.CODE

Start:

mov ax,@data ;set data segment

mov ds,ax ;initialize data segment register

mov dx,OFFSET PrnStrg ;Load dx with offset to data

mov ah,09 ;DOS call to print string

int 21h ;call DOS to print string

mov ah,4Ch ;prepare to exit

int 21h ;quit and return to DOS

END Start

We are using the ‘small’ memory model, although the ‘tiny’ model would work just as well. We’ve reserved 256 bytes for the stack space, but it is difficult to say if we’ve used any stack space at all, since we didn’t make any subroutine calls.

The data space is defined with the .data directive and we define a byte string, “Hello World$”. The ‘$’ is used to tell DOS to terminate the string printing. Borland7 suggests that instruction labels be on lines by themselves because it is easier to identify a label and if an instruction needs to be added after the label it is marginally easier to do. However, the label may appear on the same line as the instruction that it references. Labels which reference instructions must be terminated with a colon and labels which reference data objects do not have colons. Colons are not used when the label is the target in a program, such as a for a loop or jump instruction.

The reserved word, offset, is used to instruct the assembler to calculate the offset from the instruction to the label, ‘PrnStrg’ and place the value in the DX register. This completes the code that is necessary to completely specify the segment and offset of the data string to print. Once we have established the pointer to the string, we can load the AH register with the DOS function call to print a string, 09. The call is made via a software interrupt, INT 21h, which has the same function as the TRAP #15 instruction did for the 68K simulator.

The program is terminated by a DOS termination call (INT 21h with AH = 4Ch) and the END reserved word tells the assembler to stop assembling. The label following the END directive tells the assembler where program execution is to begin. This can be different from the beginning of the code segment and is useful if you want to enter the program at some place other than the beginning of the code segment.