Computer Architecture

33d  stage

9-5-2012

College of sciences for women ((())) Dept. of computer sciences

Year 2011-2012

Lec. (19)

10 Multiprocessor System

10.1 INTRODUCTION

A multiple processor system consists of two or more processors that are connected in a manner that allows them to share the simultaneous (parallel) execution of a given computational task. Parallel processing has been advocated as a promising approach for building high-performance computer systems. Two basic requirements are inevitable for the efficient use of the employed processors. These requirements are (1) low communication overhead among processors while executing a given task and (2) a degree of inherent parallelism in the task.

A number of communication styles exist for multiple processor networks. These can be broadly classified according to (1) the communication model (CM) or (2) the physical connection (PC). According to the CM, networks can be further classified as (1) multiple processors (single address space or shared memory computation) or (2) multiple computers (multiple address space or message passing computation). According to PC, networks can be further classified as (1) bus-based or (2) network-based multiple processors. Typical sizes of such systems are summarized in Table below.

TABLE 11.1 Typical Sizes of Some Multiprocessor Systems

Figure 41 Four types of multiprocessor interconnection networks. (a) Single-bus system, (b) multi-bus system, (c) multi-stage interconnection network, (d) crossbar system

The organization and performance of a multiple processor system are greatly influenced by the interconnection network used to connect them. On the one hand, a single shared bus can be used as the interconnection network for multiple processors. On the other hand, a crossbar switch can be used as the interconnection network. While the first technique represents a simple easy-to-expand topology, it is, however, limited in performance since it does not allow more than one processor/memory transfer at any given time. The crossbar provides full processor/memory distinct connections but it is expensive. Multistage interconnection networks (MINs) strike a balance between the limitation of the single, shared bus system and the expense of a crossbar-based system. In a MIN more than one processor/memory connection can be established at the same time. The cost of a MIN can be considerably less than that of a crossbar, particularly for a large number of processors and/or memories. The use of multiple buses to connect multiple processors to multiple memory modules has also been suggested as a compromise between the limited single bus and the expensive crossbar. Figure 41 illustrates the four types of interconnection networks mentioned above.

9.2 Shared Memory Organization

There has been recent growing interest in distributed shared memory systems. This is because shared memory provides an attractive conceptual model for interprocess interaction even when the underlying hardware provides no direct support. A shared memory model is one in which processors communicate by reading and writing locations in a shared memory that is equally accessible by all processors. Each processor may have registers, buffers, caches, and local memory banks as additional memory resources. These include access control, synchronization, protection, and security. Access control determines which process accesses are possible to which resources. Access control models make the required check for every access request issued by the processors to the shared memory, against the contents of the access control table. The latter contains flags that determine the legality of each access attempt. If there are access attempts to resources, then until the desired access is completed, all disallowed access attempts and illegal processes are blocked. Requests from sharing processes may change the contents of the access control table during execution. The flags of the access control with the synchronization rules determine the system’s functionality. Synchronization constraints limit the time of accesses from sharing processes to shared resources. Appropriate synchronization ensures that the information flows properly and ensures system functionality. Protection is a system feature that prevents processes from making arbitrary access to resources belonging to other processes. Sharing and protection are incompatible; sharing allows access, whereas protection restricts it. Running two copies of the same program on two processors will decrease the performance relative to that of a single processor, due to contention for shared memory. The performance degrades further as three, four, or more copies of the program execute at the same time.

A shared memory computer system consists of (1) a set of independent processors, (2) a set of memory modules, and (3) an interconnection network. The simplest shared memory system consists of one memory module (M) that can be accessed from two processors Pa and Pb (Fig. 42). Requests arrive at the memory module through its two ports. An arbitration unit within the memory module passes requests through to a memory controller. If the memory module is not busy and a single request arrives, then the arbitration unit passes that request to the memory controller and the request is satisfied. The module is placed in the busy state while a request is being serviced.

Figure 42 A simple shared memory scheme

If a new request arrives while the memory is busy servicing a previous request, the memory module sends a wait signal through the memory controller to the processor making the new request. In response, the requesting processor may hold its request on the line until the memory becomes free or it may repeat its request some time later. If the arbitration unit receives two requests, it selects one of them and passes it to the memory controller. Again, the denied request can be either held to be served next or it may be repeated some time later.

The use of different interconnection networks in a shared memory multiprocessor system leads to systems with one of the following characteristics:

1. Shared memory architecture with a uniform memory access (UMA)

2. Cache-only memory architecture (COMA)

3. Distributed shared memory architecture with nonuniform memory access (NUMA)

Figure 43 Three examples of shared-memory architectures. (a) UMA, (b) COMA, (c) NUMA

Figure 43 shows typical organization for the abovementioned three shared memory architectures. In the UMA system, a shared memory is accessible by all processors through an interconnection network in the same way a single processor accesses its memory. Therefore, all processors have equal access time to any memory location. The interconnection network used in the UMA can be a single bus, multiple bus, a crossbar, or a multiport memory.

In the NUMA system, each processor has part of the shared memory attached. The memory has a single address space. Therefore, any processor could access any memory location directly using its real address. However, the access time to modules depends on the distance to the processor. This results in a nonuniform memory access time. A number of architectures are used to interconnect processors to memory modules in a NUMA. Among these are the tree and the hierarchical bus networks.

Similar to the NUMA, each processor has part of the shared memory in the COMA. However, in this case the shared memory consists of cache memory. A COMA system requires that data be migrated to the processor requesting it.